TNT - A UNIX Packet-Radio Terminal Program

----------------------------------------------------------------------------

Introduction

Version of TNT described

License, copying, warranty

About Packet Radio and TNT

Why this program?

What's needed?

Mailbox program DPBox

1. Screen layout

1.1. Virtual screens

1.2. Types of virtual screens

1.3. Main statusline

2. Commands

2.1. Keyboard-commands

2.1.1. Cursor movement and miscellaneous

2.1.2. Window movement and control

2.1.3. Screen switching

2.1.4. Keyboard macros

2.2. Commands in command mode

2.2.1. TNC-commands

2.2.2. External commands

2.2.2.1. Saving data to files

2.2.2.2. Sending files

2.2.2.3. Shell,Run,Redirect and Sockets

2.2.2.4. Directory

2.2.2.5. Miscellaneous

2.2.2.6. Routing scripts

2.2.2.7. Extended monitor and boxlist

2.2.2.8. Interface commands

2.2.2.9. DPBox-interface commands

2.2.2.10. Password generation commands

2.2.2.11. Remote-command permissions and access levels

2.2.2.12. Broadcast transmission/receiption

3. Detailled description

3.1. Remote commands

3.1.1. Default access levels for remote-commands

3.1.2. Sysop password validation (//SYSOP)

3.1.3. Shell with root-permissions (//ROOTSH)

3.1.4. Socket connection (//SOCKET)

3.1.5. Dangerous //ECHO command

3.1.6. Handling of unknown remote commands

3.1.7. Extended remote commands

3.2. Umlaut conversion

3.2.1. Display

3.2.2. Sending text

3.2.3. File-receive

3.2.4. File-send

3.3. Using UNIX-features

3.3.1. Shell-login

3.3.2. Redirection

3.3.3. Running programs

3.3.4. Socket server

3.3.4.1. AX25-Server

3.3.4.2. Netcmd

3.3.5. Socket connect

3.4. File transfer methods

3.4.1. AutoBIN file transfer

3.4.2. YAPP file transfer

3.4.3. 7Plus file reception

3.5. Special connect text and files with macros, Name-database

3.6. Routing scripts

3.7. Call update

3.8. Logbook

3.9. Keyboard macros

3.10. Boxlist

3.10.1. General description

3.10.2. Using boxlist

3.10.3. Using keyboard macros

3.10.4. Recognized formats

3.11. Extended monitor

3.12. Use of DPBox

3.12.1. General description

3.12.2. Using unix socket interface

3.12.3. Mailbox screen

3.12.4. Using DPBox via Packet Radio

3.12.5. Autobox and monbox feature

3.12.6. Unproto list handling

3.13. Automatic password generation

3.13.1. General description

3.13.2. DIEBOX

3.13.3. FlexNet

3.13.4. TheNet

3.13.5. Baycom

3.13.6. MD2

3.14. Huffman compression

3.15. Handling Flexnet connection quality checks

3.16. Operating different software with same callsign

3.17. PACSAT broadcast operation

3.18. Autostart on connect

3.19. TNT as daemon, TNTC

4. Description of configuration files

4.1. Main configuration file

4.1.1. Serial and general configuration

4.1.2. Security

4.1.3. Directories, Files and Sockets

4.1.3.1. Directories

4.1.3.2. Files

4.1.3.3. Sockets and Boxfiles

4.1.4. Lines of virtual screens

4.1.5. Display configuration

4.1.6. Screen attributes

4.1.6.1. Color attributes

4.1.6.2. Monochrom attributes

4.1.7. Packet assembly timeout

4.1.8. Additional options

4.2. TNC-configuration files

4.3. Cookie file

4.4. Files for remote commands

4.5. Files for connect text

4.6. Files for names database and routing scripts

4.6.1. Names database

4.6.2. Routing database

4.7. User-Id's and security

4.8. Logfile for resynchronisation

4.9. File for keyboard macros

4.10. File for password generation

4.11. File for sysop authentification

4.12. File for remote commands disabling

4.13. File containing not own callsigns

4.14. File containing Flexnet digipeaters

4.15. File for AX25-server access

4.16. File for autostart on connect

4.17. File for extended remote commands

4.18. Files for BBS features

4.19. Configuration file for TNTC

5. Additional Information

5.1. Options at startup

5.2. Running under X11

5.3. Porting of TNT

6. Credits and Contact

A. Appendix

A.1. Static huffman compression table

----------------------------------------------------------------------------

Introduction

----------------------------------------------------------------------------

Version of TNT described

This documentation describes TNT V1.0 dated 97/01/26. The last update of the

documentation was done at 97/01/26.

----------------------------------------------------------------------------

License, copying, warranty

TNT is Copyright (c) 1993-1997 by Mark Wahl, DL4YBG

This program is free software; you can redistribute it and/or modify it

under the terms of the GNU General Public License as published by the Free

Software Foundation;

This program is distributed in the hope that it will be useful, but WITHOUT

ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or

FITNESS FOR A PARTICULAR PURPOSE. See the GNU General Public License for

more details (contained in file 'license').

You should have received a copy of the GNU General Public License along with

this program; if not, write to the Free Software Foundation, Inc., 675 Mass

Ave, Cambridge, MA 02139, USA.

----------------------------------------------------------------------------

About Packet Radio and TNT

Packet-Radio is a special mode used by Radio Amateurs to transfer text and

data. It is packet orientend and uses the AX.25-protocol (a special version

of X.25 to fit Ham Radio needs).

There are some single board computers which implement the AX.25-protocol and

contain the modem-interface to the radio-transceiver. The terminal or

computer with terminalprogram is connected via a RS232-interface. These

single board computers are called Terminal Node Controller (TNC).

For most of these TNCs a special software is available (WA8DED-Software or

The Firmware by NORD><LINK) which can be switched to a special protocol

(hostmode) at the RS232-interface. If hostmode is selected it is not

possible to operate the TNC using a simple terminal, a computer with a

special terminalprogram is needed.

The advantage of hostmode is that the terminalprogram can implement virtual

screens for the different connections, the command mode and monitor mode.

All these will be displayed together on a normal terminal which leads to

confusion of the operator.

TNT is such a terminalprogram (TNT stands for TNc Terminalprogram).

----------------------------------------------------------------------------

Why this program?

There are some implementations of the AX.25-protocol available for UNIX

(KA9Q, WAMPES, e.t.c.). They all use the TNC as a simple modem (KISS-mode)

and have TCP/IP protocol implemented (using AX.25). But the operator

frontend is very poor, there are no virtual screens and scroll back buffers.

So my intention was to write a program which has a powerful operator

frontend and uses the TNC not as a modem but as a Terminal Node Controller.

----------------------------------------------------------------------------

What's needed?

The program is developed for LINUX and a VT100-compatible terminal. It

further needs a TNC with WA8DED-software or The Firmware by NORD><LINK

connected to a serial port of the computer. The Firmware by NORD><LINK is

available for TNC2, all TNC2-clones and for AEA PK232 and PK88.

WA8DED-software is available for TAPR-TNC1.

If you don't have a TNC with The Firmware or you don't want to change the

Software of your TNC, TFKISS may be used. TFKISS is a program running under

Linux, which emulates a TNC with The Firmware. It only needs a TNC running

KISS or a device which behaves like a TNC using KISS. TFKISS supports

standart KISS, KISS with checksum (SMACK) and KISS with checksum used in

Flexnet-digipeaters (RMNC-KISS).

Although LINUX is the development platform a porting of TNT to other UNIXes

shall be possible because no special functions are used. Other terminals can

be used if they have at least line insert and line delete and an entry in

/etc/termcap.

----------------------------------------------------------------------------

Mailbox program DPBox

Joachim, DL8HBS has written a very powerful packet radio program called

DigiPoint for ATARI computers. It contains a packet radio host terminal,

satellite calculations and a bulletin board system.

The BBS contains almost all features of the known BBS-systems and is capable

of performing store and forward with many other systems including packed s&f

with F6FBB-BBS's. In addition it can be filled by simply monitoring the

frequency. So you will get an up-to-date mailbox without transmitting

yourself.

So I decided to try a port of the BBS-part of DP to Linux. Joachim gave me

his Pascal-sourcecode and I translated it with P2C by Dave Gillespie

(contained in most Linux-distributions) to C. P2C did a terrific job, thanks

Dave! Only some minor prework at the sources was needed and P2C created

almost executable code. An unix socket interface was added on both sides,

TNT and DPBox and the first successful tests were performed.

Many improvements were done since then, even the PACSAT broadcast

transmitter/ receiver is included now. The combination TNT and DPBox is

running currently in some german full time BBS's and performing good and

stable. For more information please consult the DPBox-Documentation.

----------------------------------------------------------------------------

1. Screen layout

----------------------------------------------------------------------------

1.1. Virtual screens

In simple other programs all commands and the data for all channels are

displayed on one screen. This is not easy to handle and often leads to

confusion of the user. Therefore TNT uses a 'virtual' screen for every kind

of data. Because only one real display is available the operator must choose

which of the several virtual screens he wants to see. He can switch between

them by using special codes or keys on the keyboard.

To see which screen and channel is active and to give global information a

statusline is displayed at the bottom of the screen.

All virtual screens (or the two parts of the connect screen) can be

configured larger than the real display on the screen. Therefore only a

small window of the virtual screen is displayed. The window can be moved by

the operator.

----------------------------------------------------------------------------

1.2. Types of virtual screens

It is possible to operate several connection on the same frequency.

Therefore a screen for every connection is implemented. The connect screen

is splitted to a part where all input is typed and a part where all received

data will be displayed. A statusline with all information about the

connection is located between these two parts. Optionally some lines

displaying the activity on the frequency can be displayed (command MONLINES,

'lines_moncon' in tnt.ini).

To change parameters or give commands to the TNC a command screen is

available. To simplify operation all commands can be entered in the connect

screens, too by using a colon (:) as the first character.

It is possible to monitor all activity of other stations on the frequency.

To display these information a monitor screen is available.

In addition there are some additional screens for special purposes. A screen

for the heard stations list, for the extended monitor feature, for the

online-help and for the box read generator. If a link to DPBox is active,

there is a screen for the box operator- console, too.

----------------------------------------------------------------------------

1.3. Main statusline

The statusline at the bottom of the screen shows the main status of the

program. It shows the screen type (connect, command or monitor) and the

current channel. In addition any open file (send or receive) on the current

channel will be displayed.

If huffman compression is active on the selected channel CONNECT is replaced

by CONN(H) and EXTMONI by EXTM(H).

In addition a 'P' will be displayed if the data output is paused, a 'I' if

insert mode is active. If the hostmode synchronisation is lost a 'S' will be

displayed. If the TNC was busy and a resend of data from the computer is

done, a 'B' will be displayed. If a routing script is active a 'X' will be

displayed.

If data is received on a channel which is not displayed the channel number

will be displayed until it is switched to the connect screen of this

channel. If a connection is active on a channel it is displayed by a '+' at

the channel position.

If a file is open on the displayed channel, a two-character file type

identifier and the filename (only last 14 charcters) will be displayed in

the statusline. Files which receive data will be displayed in the first file

field, files which transmit data will be displayed in the second file field.

The two-character file type identifier will be explained in the file command

chapter.

----------------------------------------------------------------------------

2. Commands

----------------------------------------------------------------------------

2.1. Keyboard-commands

Notes:

The combination Alt and key generates the code <ESC>key in normal console

mode. Under X the Alt-key is not supported in that way. Therefore to

activate for example the monitor screen with X you have to type first <ESC>

and then M instead of Alt-M.

(*1) : Input part of connect or mailbox screen.

(*2) : Command screen and input part of extended monitor screen.

(*3) : Monitor screen, receive part of connect, extended monitor or

mailbox screen.

(*4) : Command, boxlist, heard and help screen

(*5) : Only on monitor screen

----------------------------------------------------------------------------

2.1.1. Cursor movement and miscellaneous

CR, LF, <CNTL>M, <CNTL>J

The line from the beginning up to the cursor position is transmitted on

the current channel (*1) or will be transferred to the command

interpreter (*2). If the first character of (*1) is a colon (":"), the

line will be sent to the command interpreter (without the colon). A

colon as the first character of (*2) will be ignored and removed. If

WHOLElin is set to on, the whole line will be used and the cursor

position is ignored.

Arrow key left, <CNTL>S

Move the cursor one character to the left, if not on the start of the

line (*1,*2).

Arrow key right, <CNTL>D

Move the cursor one character to the right, if not on the end of the

line (*1,*2).

Arrow key up, <CNTL>E

Move the cursor one line up, if not on the top of the screen

(*1,*2,*4).

Arrow key down, <CNTL>X

Move the cursor one line down, if not on the bottom of the screen

(*1,*2,*4).

<CNTL>A

Move the cursor to the start of the line (for command screen and input

part of connect screen).

<CNTL>F

Move the cursor to the last non-space character of the line (for

command screen and input part of connect screen).

INSERT, <CNTL>N

Toggle insert-mode. If insert-mode is active a 'I' is displayed in the

statusline at the bottom. In normal mode all characters are

overwritten, in insert mode all characters from cursor position to the

end of the line will be shifted one position right (*1,*2).

DEL, <CNTL>H

Delete character left of cursor and move cursor one position left. If

insert-mode is active, all characters from cursor-position to the end

of the line is moved one position left (*1,*2).

<CNTL>L

The character at the cursor position will be deleted, all characters

right of the cursor up to the end of the line will be shifted one

position left (*1,*2).

<CNTL>Y

Delete all characters from the cursor position to the end of the line

(*1,*2).

<CNTL>V

The pass character. If for example you want to send cntl-T to the

connected station first type cntl-V and then a 'T'. A different

attribute of the character shows that it is a control character (*1).

----------------------------------------------------------------------------

2.1.2. Window movement and control

<CNTL>R, Arrow key up (*5)

Move the display window of the virtual screen one line up, if not on

top of virtual screen (*3).

<CNTL>C, Arrow key down (*5)

Move the display window of the virtual screen one line down, if not on

bottom of virtual screen (*3).

Page up, <CNTL>W

Move the display window of the virtual screen one page up, if not on

top of virtual screen (*3,*4).

Page dwn, <CNTL>Z

Move the display window of the virtual screen one page down, if not on

bottom of virtual screen (*3,*4).

HOME

Move to the beginning of the window (*3,*4).

END

Move to the end of the window (*3,*4).

<CNTL>P, <ALT>P, <ESC>P

Toggle stop of data-output on the current screen, only possible on

connect and monitor screen. Stop is indicated by a 'P' in the

statusline at the bottom (Monitor screen and receive part of connect

screen).

----------------------------------------------------------------------------

2.1.3. Screen switching

F1 - F9

Switch to connect screen of channel 1 to 9. If the key is pressed again

and the currently selected channel plus 10 exists, these 10 is added to

the actual channel (2 two times F1 lead to channel 11).

F10

Switch to connect screen of channel 0 (unproto channel). If pressed

again switch to channel 10, 20, ... if they exist.

F11, <ALT>M, <ESC>M

Switch to monitor screen, if in monitor screen switch back to last

selected screen.

F12, <ALT>C, <ESC>C

Switch to command screen.

<ALT>Q, <ESC>Q

Switch to connect screen of current channel.

TAB, <CNTL>I

Select a new channel, a '??' as the channelnumber in the statusline

indicates this. A number between 00 to 99 for channels 00 to 99 must be

entered. Only existing channelnumbers are accepted. If the extended

monitor is active, the channelnumber of the extended monitor channel (0

to 4) is selected instead of the connect channel.

<ALT>H, <ESC>H

Switch to help screen.

<ALT>X, <ESC>X

Switch to extended monitor screen.

<ALT>S, <ESC>S

Update heard stations list and display heard list screen.

<ALT>L, <ESC>L

Switch to box read generator (only if activated on current channel). If

already in read generator screen switch back to screen before

selection.

<ALT>B, <ESC>B

If DPBox is connected, switch to box operator console.

----------------------------------------------------------------------------

2.1.4. Keyboard macros

<ALT>0 - <ALT>9, <ESC>0 -<ESC>9

User definable function keys for either text or commands.

----------------------------------------------------------------------------

2.2. Commands in command mode

----------------------------------------------------------------------------

2.2.1. TNC-commands

Most commands in hostmode consist of one character. It is difficult to

remember the charcter for a not often needed command. Therefore more verbose

command names were created for every TNC command. But it is still possible

to use the original command characters. Only the known commands are

implemented, if you have a TNC software which has additional commands, the

command 'TNC' must be used.

More information about the commands can be found in the documentation of the

TNC software (TNC commands of NORD><LINK The Firmware 2.6a). Some of these

commands are no longer existing in newer versions (TF 2.7).

DAMAdis,"B"     : DAMA timeout

Version,"V"     : Shows version of TNC-software

CHeck,"@T3"     : Connection timer T3

Connect,"C"     : Start connection

CText,"U"       : Response text if connect is received by TNC

DIGIpeat,"R"    : Digipeat function

Disconne,"D"    : End connection

DAYTIme,"K"     : Time and date functions

Frack,"F"       : Start value of round trip timer for retry timing

FUlldup,"@D"    : Fullduplex on modem side

MAXframe,"O"    : Maximum number of outstanding packets

Monitor,"M"     : Frequency monitor functions

MYcall,"I"      : Callsign of the local operator

With MYCALL the call sign is set permanently on this

channel, with "I" only up to the next disconnect.

A permanent MYCALL can be removed by using "$" as call

sign.

Persist,"P"     : Persistance value for transmitter keyup

RESptime,"@T2"  : Delay before Info-frame will be confirmed (timer T2)

REtry,"N"       : Maximum number of retries

SLottime,"W"    : Slottime value for transmitter keyup

Txdelay,"T"     : Time after transmitter keyup to sending of data

USers,"Y"       : Number of channel open for connection

Xmitok,"X"      : Locking of transmitter

BUFfers,"@B"    : Free buffers of TNC

A1SRTT,"@A1"    : A1-value for smoothed round trip timer

A2SRTT,"@A2"    : A2-value for smoothed round trip timer

A3SRTT,"@A3"    : A3-value for smoothed round trip timer

IPOll,"@I"      : Packet length up to which I-Poll mode is used

VALcall,"@V"    : Check callsign in connect command

All verbose commands can be abbreviated, the upcase part is mandatory.

----------------------------------------------------------------------------

2.2.2. External commands

Most of the commands can be abbreviated, the upcase part of the command is

mandatory.

2.2.2.1. Saving data to files

LOGQso <filename>

A file will be openend and all transmitted and received data on the

selected channel will be saved. If no directory was specified, the file

will be stored in 'download_dir'. If 'download_dir' is empty, the

current directory is used. A CR will be translated to a LF and

control-codes will be translated to ^X. Umlaut-conversion will be done,

too. The file type ID is 'RN', receive normal.

LOGRec <filename>

Same as LOGQSO but only received data will be saved. The file type ID

is 'RN', receive normal.

LOGSnd <filename>

Same as LOGQSO but only transmitted data will be saved. The file type

ID is 'RN', receive normal.

READ <filename>

A file will be openend and all received data will be saved. If no

directory was specified, the file will be stored in 'download_dir'. If

'download_dir' is empty, the current directory is used. Only a CR to LF

translation is done. READ shall be used to receive 7PLUS files. The

file type ID is 'RP', receive plain.

READBin <filename>

Same as READ but no translation of characters is done. The file type ID

is 'RB', receive binary.

READAbin <filename>

Start receiving a file using the AutoBIN-protocol. If no directory was

specified, the file will be stored in 'download_dir'. If 'download_dir'

is empty, the current directory is used. The file type ID is 'RA',

receive AutoBIN.

LOGAbin <filename>

Same as READABIN except that at the end of transmission the transfer

statistics are only displayed, but not sent (useful for receiving

binary files from BBSs which get confused by the statistics). The file

type ID is 'RQ', receive AutoBIN, quiet mode.

READYapp [filename]

Start receiving a file using the YAPP-protocol. If no filename was

given, the name transmitted will be used, a directory contained in the

name will be removed. If no directory was specified, the file will be

stored in 'download_dir'. If 'download_dir' is empty, the current

directory is used. The file type ID is 'RY', receive YAPP.

CLose

Close the active receive-file.

LOGMon <filename>

A file will be openend and all received data on the monitor screen will

be saved. If no directory was specified, the file will be stored in

'download_dir'. If 'download_dir' is empty, the current directory is

used. A CR will be translated to a LF and control-codes will be

translated to ^X. Umlaut-conversion will be done, too. The file type ID

is 'RN', receive normal.

RDMON <filename>

Same as LOGMON but only a CR to LF translation is done. The file type

ID is 'RP', receive plain.

RDMONBin <filename>

Same as RDMON but no translation of characters is done. The file type

ID is 'RB', receive binary.

CLOSEMon

Close the active receive-file on the monitor screen.

LOGXmon <filename> (on extended monitor screen)

LOGXmon <xmon-channel> <filename> (on other screens)

A file will be openend and all received data on the current (on

extended monitor screen) or on the specified (on other screens)

extended monitor channel will be saved. If no directory was specified,

the file will be stored in 'download_dir'. If 'download_dir' is empty,

the current directory is used. A CR will be translated to a LF and

control-codes will be translated to ^X. Umlaut-conversion will be done,

too. The file type ID is 'RN', receive normal.

RDXMON <filename> (on extended monitor screen)

RDXMON <xmon-channel> <filename> (on other screens)

Same as LOGXMON but only a CR to LF translation is done. The file type

ID is 'RP', receive plain.

RDXMONBi <filename> (on extended monitor screen)

RDXMONBi <xmon-channel> <filename> (on other screen)

Same as RDXMON but no translation of characters is done. The file type

ID is 'RB', receive binary.

CLOSEXmo (on extended monitor screen)

CLOSEXmo <xmon-channel> (on other screens)

Close the active receive-file on the current (on extended monitor

screen) or on the specified (on other screens) extended monitor

channel.

APPend [ON/OFF]

Flag if data shall be appended to existing files (only valid for

LOGQSO, LOGREC, LOGSND, LOGMON and LOGXMON).

AUTOBIn [ON/OFF]

Flag if AutoBIN-receive shall start autonomously on reception of a

valid AutoBIN-Header (AutoBIN-receive is performed in quiet mode like

started with LOGABIN-command).

AUTOYApp [ON/OFF]

Flag if YAPP-receive shall start autonomously on reception of a valid

YAPP-Header.

AUTO7Pl [ON/OFF]

Flag if 7Plus-receive shall start autonomously on reception of a valid

7Plus-Header. To indicate 7Plus reception a file type ID of 'A7' will

be displayed at the bottom statusline.

2.2.2.2. Sending files

SEND <filename>

Send a file on the selected channel, a LF will be translated to CR. If

no directory was specified, the file will be fetched from 'upload_dir'.

If 'upload_dir' is empty, the current directory is used. SEND shall be

used to send 7PLUS files. The file type ID is 'TP', transmit plain.

SENDLog <filename>

Same as SEND but control-codes will be translated to ^X and

Umlaut-conversion will be done. The file type ID is 'TN', transmit

normal.

SENDBin <filename>

Same as SEND but no LF to CR translation. The file type ID is 'TB',

transmit binary.

SENDAbin <filename>

Send a file using the AutoBIN-protocol. If no directory was specified,

the file will be fetched from 'upload_dir'. If 'upload_dir' is empty,

the current directory is used. The file type ID is 'TA', transmit

autoBIN.

SENDQbin <filename>

Same as SENDAbin, but it is not waited for #OK# after transmitting the

#BIN#-header and the statistics are not transmitted. The file type ID

is 'TQ', transmit autoBIN, quiet mode.

SENDYapp <filename>

Send a file using the YAPP-protocol. If no directory was specified, the

file will be fetched from 'upload_dir'. If 'upload_dir' is empty, the

current directory is used. The file type ID is 'TY', transmit YAPP.

BReak

Abort sending of a file.

FPACLen

Maximum packet length used for file sending. If SENDLog is used, the

length is divided by two because of possible Umlaut-conversion. The

allowed range is between 20 and 256 characters. If static huffman

compression is activated the maximum value is reduced to 255

characters. Changes are global for all channels.

2.2.2.3. Shell,Run,Redirect and Sockets

SHell

Open a shell on the current channel. The call sign of the remote

station is used as a user-id. If the user-id does not exist in the

system, it depends on 'unix_new_user', whether 'remote_user' is used

instead (0) or the user-id is created (1). All received data on the

current channel will be sent to the shell, all data from the shell will

be sent on the current channel. All line feed characters sent by the

shell will be translated to carriage return . All carriage return

characters received from the remote station will be translated to line

feed prior to sending to the shell.

(This command is available only if TNT is invoked by root.)

TSHell

Same as SHELL, but no conversion of linefeed and carriage return will

be done.

ROOTSH

Same as SHELL, but a shell with superuser privileges (root) is started.

TROOTSH

Same as ROOTSH, but no conversion of linefeed and carriage return will

be done.

ENDShell

End the shell on the current channel.

RUN <program>

On the current channel the specified program will be executed. All

received data on the current channel will be used as standard input of

the program, all data sent to standard output by the program will be

sent on the current channel. Only programs contained in 'run_dir' can

be executed. All line feed characters sent by the program will be

translated to carriage return. All carriage return characters received

from the remote station will be translated to line feed prior to

sending to the program.

RUNT <program>

Same as RUN, but no conversion of linefeed and carriage return will be

done.

ENDRun

Abort execution of the program on the current channel.

REDir <device>

Redirection of input/output. All received data on the current channel

will be sent to <device>, all data received from <device> will be sent

on the current channel.

ENDRedir

End the redirection of the current channel.

SOCKCon <socket address>

The socket specified will be connected and all data received from the

socket will be transmitted on the current channel and vice versa. All

line feed characters received from the socket will be translated to

carriage return. All carriage return characters received from the

remote station will be translated to line feed prior to sending to the

socket. More information can be found in the chapter 'Detailled

Description'.

TSOCKCon <socket address>

Same as SOCKCON, but no conversion of linefeed and carriage return will

be done.

ENDSOCKC

End the socket connection on the current channel.

SOCket AXSERV <socket address>

An AX25 server will be installed on the specified socket. From now on a

connection to this socket is possible and after authentification AX25-

connections can be started (for example internet-access to packet

radio). More information can be found in the chapter 'Detailled

Description'.

SOCket AXSPEC <socket address>

Same as SOCKET AXSERV, except data from the client to AX25 will be

transmitted directly on occurence of line feed or carriage return

without waiting for 'pty_timeout'.

SOCket NETCMD <socket address> <default callsign>

A Wampes-compatible AX25 server will be started on the specified

socket. If no own callsign is given in the connect command the default

callsign is used. More information can be found in the chapter

'Detailled Description'.

ENDSock <socket address>

End the socket server on the specified socket address. All connections

to the server are disconnected.

2.2.2.4. Directory

CD <dir>

Change the working directory to <dir>. If <dir> is empty, the

HOME-directory will be used.

CWD

Displays the current working directory.

2.2.2.5. Miscellaneous

TNC <command>

<command> is sent as a command to the TNC without any conversion.

CHANnel <x> or S <x>

Switch to connect screen of channel x. If included in a command script,

no changing of screens is performed, it simply specifies the channel

used for the following commands.

CONCall <callsign>

If the updated callsign in the statusline is corrupted by a connect

message in normal text, the callsign can be restored using this

command.

CStatus

A list of all connected channels with starttime of connect and

callsigns will be displayed.

SENDCom <filename>

Execute a file containing TNC or external commands (command script).

CBell [ON/OFF/OTHER]

Beep if the state of a connection changes (connect bell). If other,

then a beep will only be generated if a station connects on a different

than the current channel.

INFObell [ON/OFF/OTHER]

Beep if a new packet is received on any channel (information bell). If

other, then a beep will only be generated if a new packet is received

on a different than the current channel.

COOKie [ON/OFF]

Send a cookie if connected from a remote station.

UMLaut [ON/OFF]

Umlaut conversion.

CONText [ON/OFF]

Send a special connect text, using macros, overrides COOKIE.

TXEcho [ON/OFF]

Echo all sent data from input part to part with received data.

PTYEcho [ON/OFF]

Echo all data received and send on a channel on which SHELL, RUN,

SOCKET or REDIR is active.

NAME <name>

Set the name of the call on the current channel to <name>, if <name> is

not specified, the stored name is displayed (only if channel

connected).

STIme

Send time and date on the current channel.

SCOokie

Send a cookie on the current channel.

TIMESET

Send time of computer to TNC.

DATESET

Send date of computer to TNC.

RESYnc

Displays the number of hostmode resynchronisations since program start

and, if applicable, some additional information concerning the problem.

LAYer3 [ON/OFF]

Enables or disables the analysis of NETROM/TheNet headers (pid CF) in

monitored frames.

HEArd [ON/OFF]

Enables or Disables the heard list. If disabled, the heard list is not

updated any longer.

KMAcro

Reload the keyboard macro file 'func_key_file' defined in the

init-file.

MSEnd <filename>

Send a file, using macros. The file is fetched from 'macrotext_dir'

defined in the init-file.

WORDWrap [ON/OFF]

Enables the wordwrap-function. An incomplete word at the end of the

line will automatically be copied to the next line.

WHOLElin [ON/OFF]

If set to on, the whole line will be transmitted, if set to off only

the part of the line up to the current cursor position.

LINelen <value>

Set the linelength at which additional input characters are ignored or

wordwrap will be executed. This value shall normally be set to 80

characters. The default value can be specified by 'input_linelen' in

the init-file.

MONLines <value>

If you like to see a part of the monitor screen on the connect screen

the number of monitor lines can be specified here. Set to 0 if you

don't want monitor lines. The default value can be specified by

'lines_moncon' in the init-file.

CONDiv <value>

Changes the input/output-lines ratio on the connect screen. For further

details see 'scr_divide' in the init-file.

XMONDiv <value>

Changes the input/output-lines ratio on the extended monitor screen.

For further details see 'xmon_scr_divide' in the init-file.

MBOXDiv <value>

Changes the input/output-lines ratio on the mailbox screen. For further

details see 'mbscr_divide' in the init-file.

COMP [ON/OFF]

Huffman compression is enabled/disabled on the current channel.

'CONN(H)' in the bottom statusline indicates an enabled huffman

compression. If no value is given the current status is returned. To

synchronize the switching to huffman compression between local and

remote station the remote command //COMP shall be used instead.

BSCRHold [ON/OFF]

If enabled, no scrolling of the displayed window will be done if the

last line of the screen buffer is not displayed (backscroll active) and

the first displayed line is still in the screen buffer.

TABExp [ON/OFF]

If enabled, a TAB-character is expanded to position the cursor on the

next tab-stop. A tab-stop is defined every 8 characters. If disabled

'^I' is displayed instead.

FREE

This command will display the number of free buffers in the TNC. The

command does not execute a '@B'-command, but it displays the value of

the last periodical polling.

SIGNon

Display version and copyright information of TNT.

LOGBOOK [ON/OFF]

This command can disable the writing to the logbook-file.

EXit

Leave TNT.

QUIT

Leave TNTC without termination of the TNT-daemon.

2.2.2.6. Routing scripts

XConnect [portheader:]<callsign>

Start of a routing script for the specified callsign. An active script

can be ended by using 'OFF' as callsign. If a routing script is active,

a 'X' in the main statusline will be displayed. If the callsign is not

found in the routing database, a simple connect is started. If a

multiport-TNC is used, a valid portheader entered with command QRG can

be used in front of the callsign to select the port where the first

connect shall be started. Using this command an automatic allocation of

a free SSID for the source call is performed. So multiple connects to

the same digipeater are possible.

QRG [<port> <frequency>] [portheader:]

With this command the operating frequency of the program is defined.

The specified frequency is used to select valid routing data out of the

routing database. <frequency> can be any string up to 19 characters.

For TNCs with one port use 0 as <port> and omit <portheader:>: QRG 0

438.300 For multiport-TNCs for each port a frequency and the portheader

must be defined: QRG 0 438.300 1: QRG 1 430.900 2: Without arguments

the current frequency definition is displayed.

LSNOTOWN

This command lists all callsigns for which an entry in 'tnt_notownfile'

was found. These callsigns are not allowed to be used as source calls

for a xconnect. So you are able to define some SSID's of your call

which TNT will not use (needed if different software is operated with

the same callsign).

LDNOTOWN

If 'tnt_notownfile' was changed while TNT is running, it can be

reloaded with this command. A reload is necessary because TNT only

reads the file at startup and copies the data to memory.

2.2.2.7. Extended monitor and boxlist

XMON [ON/OFF]

Enables or Disables the extended monitor function. If disabled,

received monitor frames are not analysed any longer.

EXTmon <call1> <call2> [<call3> <call4>] (extended monitor screen)

EXTmon <xmon-channel> <call1> <call2> [<call3> <call4>] (other screens)

On the current (extended monitor screen) or on the specified (other

screens) channel a connection shall be monitored. If only <call1> and

<call2> is specified, frames from <call1> to <call2> and <call2> to

<call1> are monitored, otherwise frames from <call1> to <call2> and

<call3> to <call4>.

EXTAmon <call1> <call2> [<call3> <call4>]

Same as EXTMON except that the next free xmon-channel will be used. The

resulting channel will be displayed as command response.

ENDEXtm (extended monitor screen)

ENDEXtm <xmon-channel> (other screens)

The extended monitor on the current (extended monitor screen) or the

specified (other screens) channel will be closed. The connection will

be no longer monitored.

EXTComp [ON/OFF]

Huffman compression is enabled/disabled on the current extended monitor

screen. 'EXTM(H)' in the bottom statusline indicates an enabled huffman

compression. If no value is given the current status is returned.

LOGBlist

On the current channel or on the mailbox screen a file is openend and

all received data is saved. The command is similar to LOGREC except a

unique filename is generated (in /tmp directory) and the file will be

removed during exit of TNT.

BLIst [<filename>]

On the current channel or on the mailbox screen the last active file is

used for the boxlist screen and feature. If the file is not closed up

to now, it will be closed. Optionally a filename can be specified. In

this case this file will be loaded into the boxlist screen.

XBList

Finish boxlist and close the boxlist screen on the current channel.

2.2.2.8. Interface commands

IFAce <socket-name>

Build up a socket connection to an external program via an UNIX-socket

<socket-name>.

ENDIFace <socket-name>

Disconnect the socket connection to an external program via an

UNIX-socket <socket-name>.

FINIFace <socket-name>

Same as ENDIFACE, except that the external program get the command to

terminate its execution.

ACTIf <socket-name>

Activate the external program on the current channel using the already

built up socket connection.

DEACtif <socket-name>

Deactivate the external program on the current channel.

SNOCONN <string>

String which will be sent, if a program was remotely activated via the

Interface and the activation was not successful.

2.2.2.9. DPBox-interface commands

ACTBox

Build up a socket connection to DPBox via an UNIX-socket specified by

'box_socket' in the init-file.

DEACTBox

Disconnect the socket connection to DPBox.

FINBox

Disconnect the socket connection to DPBox and terminate DPBox.

BOX [callsign]

Activate DPBox on the current channel using the already built up socket

connection. If a callsign is specified at the mailbox console this

callsign is used for the boxsession instead of the box callsign.

ENDBox

Deactivate DPBox on the current channel.

AUTOBOx [ON/OFF]

If activated, all connections are scanned for mail-headers. A valid

mail-header leads to saving the following mail. If the end is reached,

the mail will be sent via the interface to DPBox.

MONBox [ON/OFF]

If activated, all monitored frames are scanned for mail-headers. A

valid mail-header leads to an extended monitoring of the connection. If

no frames are lost and the mail-end is reached, the monitored mail will

be sent via the interface to DPBox.

LMONbox

All mails which are currently received using the MONBOX-feature are

displayed.

SNOBOX <string>

String which will be sent, if DPBox was remotely activated and the

activation was not successful.

SCANMBEA [<source> <destination> <own_call> <connectcall> [<timeout>]] [$]

TNT can monitor mail-beacons and if it finds your call in it, it can

activate store and forward to get your new mail. To activate this

feature you have to give source and destination of the expected

mail-beacon (which will be the callsign of your bbs and MAIL as

destination), your own call (this call is searched in the beacon data)

and a connectcall with an optional timeout. The connection is built up

using the xconnect-feature giving connectcall and timeout as

parameters. If the connection is established store and forward will

begin. The command without parameters will give the current data. If

you want to deactivate this feature, give a '$' as a single argument.

LDBOXFil

All files concerning the box configuration ('autobox_dir',

'tnt_boxender' and 'f6fbb_box') will be reread. So changes in these

files can be made without the need of leaving TNT.

ACCUIReq [ON/OFF]

If ON, unproto list requests to the callsign defined by ACCUICal will

be sent to DPBox. DPBox will then take the needed action.

ACCUICal <BBS-callsign>

If an unproto list request is received with the callsign (including

SSID) defined here, the request will be sent to DPBox. To clear the

callsign the value '$' can be used.

2.2.2.10. Password generation commands

PRIV

This command starts the password generation sequence. The command is

rejected if the channel is not connected or the callsign of the

connected station is not contained in 'tnt_pwfile'. The taken action is

dependent on the type of the password generation defined for the

callsign.

LISTPRIV

This command lists all callsigns for which an entry in 'tnt_pwfile' was

found. In addition for every callsign the type of password generation

and the file containing the password data is listed.

LOADPRIV

If 'tnt_pwfile' was changed while TNT is running, it can be reloaded

with this command. A reload is necessary because TNT only reads the

file at startup and copies the data to memory.

2.2.2.11. Remote-command permissions and access levels

REMOte [ON/OFF]

Enables or disables remote commands for ALL channels. If disabled

commands with status ALWAYS are not available.

REMAllow [ON/OFF]

Enables or disables remote commands on the current channel. If disabled

commands with status ALWAYS are available. This command is allowed only

if connected. In addition the current status is changed at a new

connect according to tnt_noremfile.

SETACC <remote-command> [NORMAL/SYSOP/ROOT/ALWAYS]

defines which access-level a remote-station needs to execute a remote-

command. NORMAL means the command is available for everyone, commands

with SYSOP status only after a successful validation with //sys. If the

root-sysop flag was set in tnt_sysfile, commands with ROOT status are

available after validation. The ALWAYS status means that this command

is available even if the connected station is contained in

tnt_noremfile.

NOACC [ON/OFF]

If set to ON, only remote stations contained in tnt_sysfile can connect

the station. All other will receive the string defined by SNOACC and

will be disconnected afterwards.

SNOACC <string>

defines the string which is sent to a station not authorized to

connect.

LISTSYS

This command lists all callsigns for which an entry in 'tnt_sysfile'

was found. In addition for every callsign the file containing the

password data is listed and if the sysop-status can be achieved after

password validation (0: not root-sysop, 1: root-sysop).

LOADSYS

If 'tnt_sysfile' was changed while TNT is running, it can be reloaded

with this command. A reload is necessary because TNT only reads the

file at startup and copies the data to memory.

LSTNOREM

This command lists all callsigns for which an entry in 'tnt_noremfile'

was found. For these callsigns remote-commands are disabled except the

commands with status ALWAYS. The disabling of remote-commands remains

until the channel is disconnected, so connecting or reconnecting to a

different station will not enable remote-commands any more. A change is

possible using the command REMALLOW.

LDNOREM

If 'tnt_noremfile' was changed while TNT is running, it can be reloaded

with this command. A reload is necessary because TNT only reads the

file at startup and copies the data to memory.

LSTFLCHK

This command lists all callsigns for which an entry in 'tnt_flchkfile'

was found. Connections from these callsigns will get no connect-text,

no remote access and will not be linked to DPBox or other programs

using the interface. In addition no entry will be included in the

logbook.

LDFLCHK

If 'tnt_flchkfile' was changed while TNT is running, it can be reloaded

with this command. A reload is necessary because TNT only reads the

file at startup and copies the data to memory.

LSEXTREM

This command lists all currently active extended remote commands, their

access level, the number of significant characters and the related

command with parameters. This data is read from 'tnt_extremotefile' at

startup of TNT.

LDEXTREM

If 'tnt_extremotefile' was changed while TNT is running, it can be

reloaded with this command. A reload is necessary because TNT only

reads the file at startup and copies the data to memory.

AUTOStrt [ON/OFF]

Enables or disables the autostart feature.

LSAUTOST

This command lists all currently active callsign/SSID combinations, for

which autostart commands are defined. This data is read from

'tnt_autostartfile' at startup of TNT.

LDAUTOST

If 'tnt_autostartfile' was changed while TNT is running, it can be

reloaded with this command. A reload is necessary because TNT only

reads the file at startup and copies the data to memory.

2.2.2.12. Broadcast transmission/receiption

SENDBC <filename>

The specified file will be sent using PACSAT broadcast protocol.

BCRQST [ON/OFF]

If set to on, broadcast requests for missing data are generated, if off

it is just waited for a retransmission.

SHPACSAT [ON/OFF]

If set to on, broadcast frames are displayed, if off, no display is

done.

DECBCAST [ON/OFF]

If set to on, broadcast-frames are decoded. If set to off, broadcast

frames are not decoded.

BCRXstat

Display of the already received or currently received files by the

broadcast receiver including some status information.

BCTXstat

Display of the files currently transmitted by the broadcast transmitter

including some status information.

----------------------------------------------------------------------------

3. Detailled description

----------------------------------------------------------------------------

3.1. Remote commands

If remote is enabled the remote station can give some commands to the

program. All remote commands start with a double slash '//'. This double

slash must be entered directly at the beginning of a line, otherwise the

command is not recognized.

The following remote commands are recognized:

//COMP on/off          Enable/disable huffman compression

//NAME <name>          Store name in database

//CSTAT                Show all active connections

//SHELL                Start a UNIX-shell session

//TSHELL               Start a UNIX-shell session without CR/LF translation

//RUN <program>        Execute a program

//RUNT <program>       Execute a program without CR/LF translation

//RUN                  Directory of all available programs

//BOX                  Start DigiPoint Box

//COOKIE               Send a cookie

//DIR <filter>         Show directory

//DIRLONG <filter>     Show directory in long format

//FREE                 Shows space on disks

//INFO                 Info about station

//HELP                 This help-information

//NEWS                 Display news about this station

//READ <file>          Read a file

//BREAK                Abort reading file

//WRITE <file>         Write file to disk

//CLOSE                End writing file

//WPRG <file>          Write file to disk using AUTOBIN-protocol

//WPRG <file> <rfile>  Same as above, but send back //RPRG <rfile>

//RPRG <file>          Read a file using AUTOBIN-protocol

//RPRG <file> <rfile>  Same as above, but send back //WPRG <rfile>

//WYAPP <file>         Write file to disk using YAPP-protocol

//RYAPP [file]         Read a file using YAPP-protocol

//VERSION              Show version of software

//ECHO <string>        Send back string

//TIME                 Send time

//RTT [timestring]     Calculate round trip timer

//RING                 Ring the bell (call for sysop)

//QUIT                 Disconnect

//DISC                 Disconnect

//SYSOP                Start sysop password validation

//ROOTSH               Start a shell with root-permissions

//TROOTSH              Same as above without CR/LF translation

//SOCKET <sock addr>   Start a connection to a socket

//TSOCKET <sock addr>  Same as above without CR/LF translation

//COMMAND <command>    Execute a valid TNT command

----------------------------------------------------------------------------

3.1.1. Default access levels for remote-commands

All remote commands are available for everyone (status NORMAL) except for

//SHELL, //TSHELL, //ECHO, //COMMAND (status SYSOP), //ROOTSH, //TROOTSH,

//SOCKET and //TSOCKET (status ROOT). The command //COMP is available if

remote is disabled on the current channel (status ALWAYS). This default

configuration can be changed by the command SETACC.

----------------------------------------------------------------------------

3.1.2. Sysop password validation (//SYSOP)

If the remote-command //sys is received, a sysop password validation

sequence is started. TNT will pick 5 random numbers with values up to the

length of the stored password string for the remote station. The remote

station must answer with the corresponding 5 characters. The characters can

be contained at every position inside a random string to make a hacking of

the password more difficult. In addition there is no response if the //sys

command was successful or not, the status is changed only internally.

Therefore it is advisable to send //sys 3 or more times with only one

correct answer. This will make the hacking of the password quite impossible.

The algorithm used is compatible to the TheNet password validation.

----------------------------------------------------------------------------

3.1.3. Shell with root-permissions (//ROOTSH)

This command is very dangerous, please be sure that it is available only for

the right people after password validation. If you don't want to allow this

command, set it to status ROOT and set the root-sysop flag in tnt_sysfile

for all callsigns to 0.

----------------------------------------------------------------------------

3.1.4. Socket connection (//SOCKET)

This command allows to connect to every socket in your system. Therefore it

is quite dangerous, too. Change to a different status as ROOT is not

recommended. If you want to allow the connect to specific sockets, use the

extended remote commands ('tnt_extremotefile').

----------------------------------------------------------------------------

3.1.5. Dangerous //ECHO command

There are some pirat bulletins floating around in the BBS-network which use

the remote-command //ECHO to let you send a bulletin with offending

contents. Therefore I changed the access level of //ECHO to SYSOP level. If

you put all the BBS's your are working with in tnt_noremfile or if you are

using command REMALLOW to disable the remote commands on the channel you are

connected to a BBS, you can change back the access level of //ECHO to

NORMAL. But be very careful with it! Otherwise there may be an offending

bulletin forwarded worldwide with YOUR callsign as sender!

----------------------------------------------------------------------------

3.1.6. Handling of unknown remote commands

To easily add new remote commands an unknown remote command will be used as

a parameter for //RUN. For example the command //FOOBAR will be translated

to //RUN FOOBAR. If the program FOOBAR is existing in 'run_dir', it will be

executed. If the program is not existing, the user will be informed about an

invalid remote command.

----------------------------------------------------------------------------

3.1.7. Extended remote commands

The behaviour described in the last section allows an easy extenstion of the

available remote commands by providing programs in the 'run_dir'. If an

extension is needed, which cannot be implemented with a program or the

program needs specific parameters, the extended remote commands feature can

be used.

A new extended remote command consists of a valid remote command plus

parameters. In addition the access level and the number of significant

characters is defined. All data is stored in 'tnt_extremotefile'.

The contents of this file can be displayed by command LSEXTREM. When the

file was updated, it can be reread using command LDEXTREM.

----------------------------------------------------------------------------

3.2. Umlaut conversion

----------------------------------------------------------------------------

3.2.1. Display

If the LINUX-console control codes are used, for a display on the screen the

IBM-characterset is used. No umlaut-conversion will be done. If termcap is

used a received IBM-umlaut is converted depending on UMLAUT. If UMLAUT is

on, the IBM-umlaut is converted into the 8-bit Latin-1 umlaut. If UMLAUT is

off, it is converted to a two character representation ("ae").

----------------------------------------------------------------------------

3.2.2. Sending text

If UMLAUT is off, all entered umlauts are converted to a two character

representation ("ae"). If UMLAUT is on, umlauts will not be converted and

sent as an IBM-umlaut.

----------------------------------------------------------------------------

3.2.3. File-receive

Only log-files (LOGREC, LOGSND, LOGQSO, LOGMON and LOGXMON) are affected by

umlaut conversion. If UMLAUT is on, the IBM-umlaut is converted into the

8-bit Latin-1 umlaut. If UMLAUT is off, it is converted to a two character

representation ("ae").

----------------------------------------------------------------------------

3.2.4. File-send

Only log-files (SENDLOG) are affected by umlaut conversion. If UMLAUT is on,

the 8-Bit Latin-1 umlaut is converted into the IBM-umlaut. If UMLAUT is off,

it is converted to a two character representation ("ae").

----------------------------------------------------------------------------

3.3. Using UNIX-features

----------------------------------------------------------------------------

3.3.1. Shell-login

TNT allows a remote user to log into UNIX as a normal user. All received

data is treated as shell input, all data from the shell is transmitted to

the remote station. To use the shell-login it is necessary to invoke TNT by

the superuser root. Otherwise the shell-login is disabled. The shell can be

started by the operator using command SHELL or TSHELL or by the remote

station using command //SHELL or //TSHELL. It is possible to execute all

programs which are permitted to use from the shell.

To be in line with the end of line convention used in Packet Radio, all line

feed characters from the shell will be translated to carriage return and all

received carriage return characters will be translated to line feed. This

translation can be disabled by using the command TSHELL instead of SHELL or

//TSHELL instead of //SHELL as a remote command.

At login time, it is checked, if the callsign of the remote station is a

valid user-id. If it does not exist, depending on 'unix_new_user' in the

initfile a new user-id is created or the user-id specified by 'remote_user'

is used. In all cases the callsign is stored on the environment variable

'CALLSIGN'.

For some configurations it is useful to be able to get a shell with root

permissions. This can be done using the command ROOTSH and remote command

//ROOTSH or command TROOTSH and remote command //TROOTSH for disabled

CR/LF-translation. As giving superuser access to the system is very

dangerous, extra care must be taken.

To increase performance, all data will be buffered. This means that data is

not sent directly, but if the buffer contains 256 Bytes (the AX25 maximum

packetlength) or if for a specific time no new characters were received.

This time ('pty_timeout') can be configured in the initialisation file.

----------------------------------------------------------------------------

3.3.2. Redirection

A connect screen of TNT does not accept any of the standart control

characters used for screen oriented programs. If you use an AX25 connection

to work with such a program (for example using a TSHELL on another stations

TNT) you can use redirection (command REDIR) to one of the Linux virtual

consoles. Now all control codes are handled correctly and you can work with

the program like you were directly connected to the remote system.

REDIR allows to redirect all data of an AX25-channel to any device which is

known in the system. Examples are the Linux virtual consoles /dev/ttyX or

one part of a tty/pty-pair.

There is no translation of the characters, all data received is given

transparently to the device and vice versa. Like for the shell-login all

data will be buffered before transmission.

----------------------------------------------------------------------------

3.3.3. Running programs

For users who are not familiar with UNIX, the use of a shell is quite

complicated. Therefore a RUN command is implemented, which will execute a

specified program using a shell. If you don't like running TNT under

root-permissions and therefore the shell-login is disabled, the RUN command

allows to execute specific programs by a remote station.

A special directory 'run_dir' contains all executable programs. Programs in

other directories cannot be executed by this command. Because no login is

performed, the program is executed using the default user specified by

'remote_user'. If TNT was invoked not by the superuser root, the user can't

be changed. In this case the program is executed under the permission of the

userid from which TNT was started. The callsign of the user is stored in the

environment variables 'CALLSIGN' and 'LOGNAME'. The callsign including the

SSID will be available in the environment variable 'CALLSSID'.

In most cases it is wanted to translate the UNIX-linefeed to Packet-Radio

carriage return and vice versa. If no translation of the characters sent and

received by the program shall be done, the command RUNT must be used. Like

for the shell-login all data will be buffered before transmission.

----------------------------------------------------------------------------

3.3.4. Socket server

If your system is used by several people or is part of a network, you may

want to give access to Packet Radio to these users. Or you want to use

programs on your system, which must be able to perform an outgoing

AX25-connect. To handle these requirements the socket server feature

(command SOCKET) was implemented.

There are three type of servers, AXSERV, AXSPEC and NETCMD. AXSERV and

AXSPEC are almost identical, both define an AX25-server. AXSPEC does not use

the usual buffering method for data described for shell-login, but sends

data directly on occurence of a line feed or carriage return character.

NETCMD is a Wampes-compatible server which allows the usage of programs

designed for Wampes (like conversd) together with TNT.

All types of servers need a socket address. This socket address can have

different formats.

a. UNIX-sockets

The format for a UNIX-socket is 'unix:<socket path>' or 'local:<socket

path>'. The path can be an absolute path beginning with a '/' or a

relative path to 'tnt_dir'.

Example:

unix:/tcp/sockets/convers

b. INET-sockets

The format for a INET-socket is 'ip-address:port'. ip-address can be a

hostname, an ip-address or a '*' for any ip-address. Port can be any

valid port number or a name for a service.

Example:

*:3600

199.199.10.10:ftp

foo.bar.com:2000

3.3.4.1. AX25-Server

A local user can connect to this server with 'telnet localhost

<portnumber>'. Then a login with callsign and password is executed. After a

successful login the user is connected to the AX25-server and may use one

channel of the TNC for Packet Radio connections. The usage is quite simple,

help for the user is available. The login information is kept in

'sock_passfile' and is independent of the passwd file.

3.3.4.2. Netcmd

The Netcmd-server works compatible to Wampes. So after connect to the

socket, the server is in command mode and accepts three commands: ASCII,

BINARY and CONNECT. Any other input or wrong arguments lead to a closing of

the connection.

ASCII selects a translation from line feed to carriage return before

transmitting data on the AX25 side and vice versa. This is the default mode.

BINARY selects a transparent connection without any character translation.

CONNECT starts an AX25 connection on a free channel. It needs additional

parameters, the syntax is:

CONNECT <transport mode> <destination callsign> [source callsign]

The only valid value for 'transport mode' is AX25, other modes will lead to

a closing of the connection. The destination callsign must not contain any

digipeaters. TNT uses the xconnect-feature to build up the connection,

therefore the path must be defined in the routing database. Normally the

default callsign entered with the SOCKET command will be used. If a source

callsign is given in the CONNECT command, this will be used instead. There

will be an automatic SSID-update to allow several connections with the same

destination.

After a successful link setup the server will switch to data mode, all

received data will be sent to the socket, all data from the socket will be

transmitted on the AX25 side.

When the link setup was not successful, the socket connection will be simply

disconnected without any further information.

----------------------------------------------------------------------------

3.3.5. Socket connect

You may have installed some socket servers on the system. To give Packet

Radio users access to these, the socket connect feature is available. The

only Argument needed is the socket address of the socket which shall be

connected. The socket address uses the same syntax as the socket server.

Like for the other UNIX-features there is an option to translate carriage

return into line feed and vice versa (command SOCKCON, remote command

//SOCKET) or the possibility for a transparent connection (command TSOCKCON,

remote command //TSOCKET).

Because there is no access restriction to a socket, you have to be careful

with this command. A full connectivity of all sockets shall be allowed only

for a sysop. Users shall get only specific sockets by defining extended

remote commands ('tnt_extremotefile').

----------------------------------------------------------------------------

3.4. File transfer methods

----------------------------------------------------------------------------

3.4.1. AutoBIN file transfer

To transfer binary files easily without much overhead but with a security

check, the AutoBIN-'protocol' was defined. It is implemented in many

PR-programs. Therefore it is implemented in TNT, too. To use AutoBIN the

commands SENDABIN, READABIN and LOGABIN are available for the operator and

//WPRG and //RPRG for a remote user. In addition with AUTOBIN the protocol

is autonomously started after reception of a valid AutoBIN-header.

At the end of a successful transfer the elapsed time and the effective

Baudrate is displayed. If a file was received, the received checksum and the

calculated checksum will be displayed, too. Normally these statistical

information will be send to the remote station, too. In case of LOGABIN or

AUTOBIN enabled the statistical information will only be displayed (to avoid

confusion of some BBSs).

If the transfer was aborted, the connection was disconnected or the

calculated checksum is not equal to the received checksum, the received file

will be moved to a special directory (abin_dir). In addition the name is

changed to a unique name. From time to time it is needed to clean up this

directory. Although in most cases these corrupted files are of no interest,

for the rare cases where they are needed they are kept in this directory.

----------------------------------------------------------------------------

3.4.2. YAPP file transfer

The AutoBIN-'protocol' is used widely in Germany. The rest of the world

usually use the YAPP-protocol. From the technical point of view YAPP and its

extension YAPP-C is the better approach to binary transfer and shall be

preferred.

In TNT for YAPP the commands READYAPP and SENDYAPP are available, a remote

user can use //RYAPP and //WYAPP. To enable the automatic reception of a

YAPP-file, you can set AUTOYAPP to on. All files automatically received will

be stored in 'yapp_dir' defined in tnt.ini.

----------------------------------------------------------------------------

3.4.3. 7Plus file reception

If you receive large numbers of 7Plus files, you might want to store them

directly in a special directory (without any character before and after).

This can be done by setting AUTO7PL to on. Now all received 7Plus files will

be stored in 'tnt_7plus_dir' defined in tnt.ini.

----------------------------------------------------------------------------

3.5. Special connect text and files with macros, Name-database

To be able to send a special connect text to a connecting station, macros

can be included in the connect text file. The connect text file will be sent

on a connect if CONTEXT was set to ON. For default the connect text file

(tnt_ctextfile) in the tnt directory will be used, but you can put for some

users a personal connect text in the connect text directory (ctext_dir). The

file must be named <callsign>.ctx .

In addition a file containing macros can be sent every time you like, not

only on connect (command MSEND). All these files must reside in the macro

text directory 'macrotext_dir'.

To be more personal and to help remembering the name of the operator of the

remote station a name database is included. A name can be entered by the

operator using the NAME command or with //NAME given by the remote station.

If no name is specified after the command, the current name will be

reported.

Following macros are expanded, if found in a connect text or macro file:

%v: Version of TNT running

%c: Call of other station

%n: Name of other station (extract from names database)

%y: Call of own station

%k: Channel number

%t: Current time

%d: Current date

%b: Bell (^G)

%i: Include news-file ('news_file_name')

%z: Timezone

%_: CR/LF

%o: Include a cookie

%?: Send a message if name of other station not contained in

names database

%%: '%' itself

----------------------------------------------------------------------------

3.6. Routing scripts

The structure of the Packet Radio Network is quite different compared to the

internet. In the internet you can directly specify your destination and the

routing will be done automatically. In the Packet Radio Network there exist

different systems with incompatible routing mechanisms. To connect to a

destionation there may be needed several steps. Suppose the following

example:

You want to connect DL7ZZZ which is standby on the frequency of the

digipeater DB0LUC. Your local digipeater is DB0BLO. Therefore your first

command is to connect DB0BLO:

:c db0blo

If the link setup was successful you will send the line

c db0ber

to the digipeater which then will build up a connection to DB0BER, which is

the next digipeater towards DB0LUC. If the connection is established, DB0BLO

will send you the message:

*** connected to DB0BER'

Now you enter the line

c db0luc db0bln

which means DB0LUC can be reached from DB0BER via DB0BLN. DB0BLN is not

connected, because it uses hop-to-hop acknowledge instead of simple

digipeating. Now if the connection was successful, you get the line

BSUED:DB0BER> Connected to DB0LUC via DB0BLN

and you can at last try to connect your destination with the line:

c dl7zzz

The successful connection is indicated by the line

*** connected to DL7ZZZ

If you do this manually, you must look at your screen and after each

successful connect-message you must enter the new line. The connect-script

facility (command XCONNECT) now does all these steps automatically, if you

have given the routing information to the program. The routing information

will be searched in the routing-database file (route_file_name) and reads

for the example as follows:

T>DL7ZZZ Joe; N>DB0LUC T>DL7ZZZ

N>DB0LUC Digi Luckau; N>DB0BER F>DB0BLN N>DB0LUC

N>DB0BER Digi Tempelhof; N>DB0BLO N>DB0BER

N>DB0BLO Digi 9K6

As you can see the routing entries are recursive, you don't have to specify

the whole routing for the destination. The characters before the callsigns

give the information to the connect script shall connect the digipeater ( N>

) or if the digipeater shall be used in the via path ( F>, D> ). To identify

a normal user T> is used, for mailboxes B> can be used.

Some programs used by normal users allow a connect with the command //c

<callsign>. If you want to use such a user as part of your path, put a T> in

front of the callsign and a '//c <callsign>'-line will be send by the

routing script.

If you frequently change your operating frequency, you may want to use the

QRG command and want to define routing data only valid for a special

frequency ('<IF xxxx> <END>'-clause).

----------------------------------------------------------------------------

3.7. Call update

If you use routing scripts or if you connect manually through a large number

of digipeaters, you easily loose the orientation, where you are. Therefore,

every 'connected to' line will update the callsign displayed in the

statusline.

'reconnected to' lines will update the call in the statusline, too, but

routing scripts will not send the next command line and the logbook call

will not be updated.

If the callsign is changed by a 'connected to' string in normal text, it can

be restored by using the CONCALL command.

----------------------------------------------------------------------------

3.8. Logbook

All connections performed by the station are written into a logbook-file.

Each line contains starttime and endtime of the connection and the callsign

of the remote station. If the destination was connected directly only this

call is logged, if the destination was connected using several digipeaters

or by a routing script, the first digipeater used is logged, too. The name

of the logfile can be specified in the init-file (tnt_logbookfile). The

command LOGBOOK can be used to enable or disable the logbook function.

Example:

Starttime   |    Endtime     |          Callsign

------------------------------------------------------------------

16.03.94 18:08 | 16.03.94 18:09 | DB0BLO

17.03.94 21:32 | 17.03.94 21:37 | GEHREN:DB0LUC, Uplink: DB0BLO

----------------------------------------------------------------------------

3.9. Keyboard macros

With the keyboard macro feature some often used commands or texts can be

executed or sent by only one keycode. You may define up to 10 keycodes

(<ALT>0 to <ALT>9 or <ESC>0 to <ESC>9) with either text patterns or TNT

commands in a file specified by 'func_key_file' in the init-file.

Each line has to begin with a two digit number specifying the keycode and a

colon. (01: for <ALT>1 / <ESC>1, 10: for <ALT>0 / <ESC>0). A text line must

follow directly after the colon. If the last character of the line is an

asterisk '*' a CR will be sent instead of it, otherwise no CR will be sent.

A command must be preceeded by an additional colon, a CR is always appended

to the command line. The length of the text or command must not exceed the

length of the line.

If the length of one line is not sufficient for your text or you want to

send a text containing macros like time and callsign, you must specify the

commands 'send' or 'msend' instead of a textline. The keyboard macro file is

loaded at startup. If you change this file during execution of TNT, you can

reload the file with the command 'kmacro'.

----------------------------------------------------------------------------

3.10. Boxlist

----------------------------------------------------------------------------

3.10.1. General description

If you get a list of files from a BBS, you will have to write down the files

you are interested in or you must store the list in a file and look through

this file afterwards. In both cases you have to type the filename or number

of the file to get it from the BBS.

The boxlist-feature allows to fetch a file from the BBS by selecting the

file (with cursor keys) from the filelist and simply pressing CR.

If an Interface-connection to a DPBox is active and a 'check'-command with

bulletin-ID in the list is given, all bulletins which are in the bulletin-ID

pool of the DPBox are displayed with a special attribute/colour. That means

you get an overview about the bulletins already contained in your own BBS.

----------------------------------------------------------------------------

3.10.2. Using boxlist

How does it work?

a. Before requesting the filelist open a logfile. If you are not

interested in keeping the list, simply use the command 'logblist'. A

temporary file with a unique name will be used. This file will be

erased when you exit TNT. If you want to keep the list, use 'logrec

<filename>'.

b. If the end of the filelist is received, use the command 'blist'. This

command closes the logfile and loads it into the boxlist screen. If no

file was open the last logfile on the current channel will be loaded

into the boxlist screen.

c. Use <ALT>L or <ESC>L to toggle connect and boxlist screen. In boxlist

screen you can use the cursor commands to select a file. Pressing CR

will send a read command to the BBS. All already fetched files will be

displayed with a different attribute.

d. If you have read all files from the filelist you can close the boxlist

screen by the command 'xblist'.

----------------------------------------------------------------------------

3.10.3. Using keyboard macros

To use the boxlist feature without typing in the commands, it is advisable

to use keyboard macros. For example:

<ALT>8 / <ESC>8 : 'logblist'

<ALT>9 / <ESC>9 : 'blist'

<ALT>0 / <ESC>0 : 'xblist'

These keyboard macros are already included in the sample keyboard macro

file.

----------------------------------------------------------------------------

3.10.4. Recognized formats

At the moment the following list formats are recognized:

a. DIEBOX check

7 DL4BCU > TERMINE...16 24.09.94 DL 2214 5 2m Mobilfuchsjagd I05 08.

-> R TERMINE 16

b. DIEBOX list

263 DL1ZAX 02.11.94 18:03 6763 DL-RUNDSPRUCH NR. 39/94

-> R 263

c. DIEBOX checklist with BID

85 DH3FBI > KENWOOD..423 055514DB0GV DL 851 5 LF & VLF Empfang mit TS-5

-> R KENWOOD 423

d. RUN C with option D=CRD$@L

DG0XC DIGI......17 28.04.95 2845DB0BALWE DL 1 DB0BRO-1 wieder ok.

-> R DIGI 17

----------------------------------------------------------------------------

3.11. Extended monitor

The extended monitor feature allows the monitoring of specific connections

on the frequency with automatic removal of headers and resent packets. There

are 5 (0 to 4) different extended monitor channels available. <ALT>X or

<ESC>X will switch to the extended monitor screens and TAB can be used to

change the channel.

The command EXTMON is used to activate the extended monitor. If you have

given the command from an extended monitor screen, the current channel will

be taken. On other screens you must give the channel number as an additional

parameter before specifying the callsigns. The next free extended monitor

channel will be taken if the command EXTAMON is used.

As command parameters you have to specify the callsigns of the connection

you want to monitor. Normally this will be 2 callsigns and both directions

will be monitored (in different attributes).

Using digipeaters like NETROM/TheNet or RMNC/Flexnet a connection is build

up out of 2 connections. First the connection from station 1 to the

digipeater (DL9xxx <> DB0xxx) and second the connection from the digipeater

to station 2 (DL9xxx-15 <> DG1xxx). In most cases you can only monitor

frames sent by the digipeater. In this case you can specify 4 callsigns

(DB0xxx DL9xxx DL9xxx-15 DG1xxx) and both directions of the connection will

be monitored in the extended monitor screen.

If the monitored connection is huffman-coded, decoding can be activated by

command

EXTCOMP.

The extended monitoring can be finished by the ENDEXTM command.

----------------------------------------------------------------------------

3.12. Use of DPBox

----------------------------------------------------------------------------

3.12.1. General description

DPBox is a daemon which is totally independent of TNT. The daemon must be

activated before using any of the DPBox-commands of TNT.

----------------------------------------------------------------------------

3.12.2. Using unix socket interface

To build up the connection from TNT to DPBox the command 'actbox' must be

used. If the connection shall be closed 'deactbox' must be used.

The command 'finbox' not only closes the connection like 'deactbox', it

gives a termination command to the DPBox daemon.

The unix socket name of DPBox must be defined in 'box_socket' in the

initfile.

----------------------------------------------------------------------------

3.12.3. Mailbox screen

If the connection to DPBox is activated, the mailbox screen (<ALT>B or

<ESC>B) can be used as a sysop-console by executing the 'box' command.

You can either 'quit' or give the command 'endbox' to end your sysop-

session.

All features of a normal connect screen are provided including the boxlist.

In boxlist mode there is an additional feature. If you select a file and

press 'e' instead of CR, the file will erased. 'k' for kill and 'l' for list

are available, too. 't' generates a transfer line where you can add an new

board name, '0' sets the lifetime of the message to 0.

----------------------------------------------------------------------------

3.12.4. Using DPBox via Packet Radio

DPBox can be activated on a normal connect channel by the command 'box', by

the remote command //box or by connecting a channel which SSID of the

callsign is equal to 'tnt_box_ssid' in the initfile.

DPBox is activated only, if the channel is connected.

The 'endbox' command, 'quit' to the box or a disconnect will terminate the

box-session.

----------------------------------------------------------------------------

3.12.5. Autobox and monbox feature

If you set 'autobox' to on, all mails you will receive on any channel will

be sent to DPBox and will be stored in the corresponding board. This

includes personal mail and bulletins.

If you set 'monbox' to on and 'xmon' is on, all monitored frames will be

searched for mail headers. If one is found, it is tried to receive the whole

mail similar to the extended monitor function. If packets are lost or if any

other error occurs, the mail is not accepted. If the mail was correctly

received it will be sent to DPBox and will be stored in the corresponding

board. All bulletins and all personal mail which contains a callsign out of

'autobox_dir' as sender or receiver will be treated in this way. All mails

which will currently be received can be displayed with the 'lmonbox'

command.

----------------------------------------------------------------------------

3.12.6. Unproto list handling

Unproto list handling, a feature know from F6FBB-BBS's,is now available

together with DPBox 5.03.00 upwards. The BBS will send an unproto frame for

every newly arrived mail. This frame contains a message number which can be

used for a mail request by an user programm like TPK or TSTHOST.

If some unproto frames were lost, a listening station can request a resend

of older messages. To send these requests to DPBox, you have to activate the

feature using command ACCUIREQ und you have to define the BBS's callsign

with command ACCUICAL. This callsign must be equal to the source-callsign of

the unproto frames the BBS is sending.

All these configurations are needed to enable unproto list handling in a

real BBS. The other direction, TNT handling unproto lists as a client like

TPK or TSTHOST is not implemented up to now.

----------------------------------------------------------------------------

3.13. Automatic password generation

----------------------------------------------------------------------------

3.13.1. General description

In the packet radio network many different password systems are used at the

moment. Therefore a configuration file 'tnt_pwfile' is provided. In this

file for any callsign including SSID a password type and a file containing

the password can be defined. Depending on the password type a different

action is taken and the contents of the password file are different.

Up to now only few password systems are implemented in TNT, but it is

planned to increase the number of supported systems. If you have added any

password system, please contribute!

----------------------------------------------------------------------------

3.13.2. DIEBOX

If the password type is DIEBOX, after a connect to the specified callsign

was done, every text is scanned for the string 'Login: '. If the string was

found, the following date and time is recorded and the scan for the string

will be stopped.

If you invoke the command 'priv', a 4 character password will be generated

out of the logintime and your password file and as a result 'PRIV xxxx' will

be sent (xxxx stands for generated password).

The password may contain CR and LF, CR, LF or none of both. TNT will detect

this from the length of the file.

----------------------------------------------------------------------------

3.13.3. FlexNet

If the password type is FLEXNET and the connection to the wanted station is

established, the command 'priv' starts the password sequence. A string

containing 'SYS' is sent to the remote station which will respond with a

string like '(xyz) abcde>'. An answer is calculated out of this response

together with the specified password. This answer is sent to the remote

station and sysop-status will be available.

Unlike the other password systems the third value in the configuration file

is not the filename where the password is stored, but directly the password

number.

There is some other software which uses the same password algorithm as

FlexNet, but need a different activation string than SYS. In this case a

fourth (optional) value can be used to define this activation string.

----------------------------------------------------------------------------

3.13.4. TheNet

If the password type is TheNet and the connection to the wanted station is

established, the command 'priv' starts the password sequence. A string

defined by the fifth value in the configuration file is sent to the remote

station which will respond with a string like 'NODE:NO1DE: a b c d e' or

simply 'a b c d e'. This response defines which characters out of the

password file must be sent as an answer. This answer is generated and is

sent to the remote station and sysop status will be available.

The third value in the configuration file defines the file where the

password is stored.

The fourth value in the configuration file defines if some additional

features shall be used. If Bit 0 is set, the password generation is done 3

times, but only one ramdomly selected time the password generation is

correct. The other two 2 times a random string is sent instead of the right

answer. If Bit 1 is set, the answer is hidden in a 72 character string. If

Bit 2 ist set, perfect hiding is used. This means that only characters out

of the password string will be used to generate the line. Alternatively you

can add a second line to the password file. In this case only characters out

of this line will be used.

The TheNet-type password generation is used by other software, too (for

example Baycom, DigiPoint).

----------------------------------------------------------------------------

3.13.5. Baycom

The Baycom password system is almost identical to TheNet, except that no

activation string is needed and right after connect you will receive the 5

random numbers.

The syntax is the same as for TheNet, except the fifth parameter (the

activation string) can be omitted. The default will be SYS. Because the

string is needed only if you defined a special configuration for the user

profile in BayBox, it is normally not of any importance.

There is another small difference to the TheNet syntax, the option '3 tries,

where only one answer is correct', is not allowed.

----------------------------------------------------------------------------

3.13.6. MD2

After connect a similar like the following string will be received:

XXXBBS> [ABCDEFGHIL]

Out of this string an answer is generated, the password algorithm used is

'RSA Data Security, Inc. MD2 Message Digest Algorithm'.

----------------------------------------------------------------------------

3.14. Huffman compression

The huffman data compression implemented in some other programs is

available, too. It can be activated by the remote command //COMP or via

command COMP. The compression will lead to a data reduction of about 30% for

plain ASCII text. For binary files the compression is not useful.

To read a QSO in extended monitor which uses huffman compression the command

EXTComp is provided.

The MONBOX feature detects huffman compression and will in most cases catch

mails even if they are read in //COMP mode.

Compression is available only if tnt_comp in the configuration file is set.

In this case the maximum allowed packet length (file_paclen in configuration

file, command FPACLEN) for file sending is 255 Bytes.

The //comp - method was first used by DK4NB in SP 9.00, the translation

table is his setup.

----------------------------------------------------------------------------

3.15. Handling Flexnet connection quality checks

The Flexnet digipeater protocol uses connects to determine the connection

quality and availability. These connects leads to unnecessary activation of

connect-texts, DPBox or other programs connected at the interface. In

addition the logbook will be flooded with these connections.

So TNT provides a file where the callsigns of Flexnet-digipeaters performing

these connection quality checks can be stored (tnt_flchkfile). If a callsign

found in this file is connecting, it will get no connect-text, no remote-

access and no connection via interface will be started. The logbook will not

contain this connection.

The contents of this file can be displayed by command LSTFLCHK. When the

file was updated, it can be reread using command LDFLCHK.

----------------------------------------------------------------------------

3.16. Operating different software with same callsign

If you want to operate different software with the same callsign, you want

to define what SSID's of your call TNT must not use. This can be done in the

file defined by 'tnt_notownfile'. Calls contained in this file will not be

used for connecting using routing scripts.

The contents of this file can be displayed by command LSNOTOWN. When the

file was updated, it can be reread using command LDNOTOWN.

----------------------------------------------------------------------------

3.17. PACSAT broadcast operation

TNT includes a PACSAT broadcast compatible receiver and transmitter. The

code is mainly based on the pascal source code written by Joachim, DL8HBS.

The receiver is enabled by setting DECBCAST to ON. All files received in

frames in PACSAT broadcast protocol will be decoded. Depending on the type

of the file, a complete file will be stored in 'tnt_bcsavedir' (normal

files) or will be sent to DPBox (BBS-files).

If parts of the received file are missing and the broadcast-transmitter

allows it by a not permanent transmission, the missing parts can be

requested. This is enabled by setting BCRQST to ON.

The status of the broadcast receiver can be displayed by BCRXSTAT. All files

successfully received or currently in receiption will be displayed. Some

statistical information shows the progress.

The broadcast transmitter is controlled either by DPBox to do a broadcast of

BBS-files. More information can be found in the DPBox-documentation. In

addition a broadcast transmission of normal files (command SENDBC) is

possible, too.

The status of the broadcast transmitter can be displayed by BCTXSTAT. All

files currently transmitted are listed including some statistical

information.

As the TheFirmware and WA8DED-software does not allow to change the PID used

for transmitted frames, the broadcast transmission will be sent using PID

F0. This can lead to problems using other software (WISP) as a receiver.

TFKISS and a special version of TheFirmware (ask me or DL8HBS for it) allows

the change of the PID and therefore the correct broadcast transmission

according to the protocol.

----------------------------------------------------------------------------

3.18. Autostart on connect

There are many possibilities in TNT (RUN, SOCKET) to allow a remote user to

use other software on the system. But to start these applications you first

have to give a special command when connected to TNT.

To make these application easily accessible and to hide that TNT is doing

the work, you can define for specific callsign/SSID combinations an

autostart command. This command, which contains any valid remote command

plus parameters, will be executed, when the callsign/SSID combination will

be connected.

The autostart-feature must be enabled using command AUTOSTRT, by default

autostart is disabled.

To be able to connect the callsign/SSID combination, you have to define one

or more channels of your TNC to use this callsign (use MYCALL in tnt.up).

The callsign/SSID combinations and the autostart commands are defined in

'tnt_autostartfile'. The actual data can be displayed using LSAUTOST, if the

file was changed, it can be reread using LDAUTOST.

----------------------------------------------------------------------------

3.19. TNT as daemon, TNTC

Normally, TNT will use a console for the user interface. But there are some

cases where this is not wanted or needed:

- TNT user interface needed on different host

- changing between X and terminalmode without terminating TNT

- no user interface needed

Therefore TNT can be started as a daemon using the command line parameter

'-d' on startup. TNT then accepts socket connect requests on the socket

address specified after 'frontend_socket' in tnt.ini. This socket address

can have different formats (equal to the definition for socket servers):

a. UNIX-sockets

The format for a UNIX-socket is 'unix:<socket path>' or 'local:<socket

path>'. The path can be an absolute path beginning with a '/' or a

relative path to 'tnt_dir'.

Example:

unix:tntsock

unix:/tcp/sockets/convers

b. INET-sockets

The format for a INET-socket is 'ip-address:port'. ip-address can be a

hostname, an ip-address or a '*' for any ip-address. Port can be any

valid port number or a name for a service.

Example:

*:3600

199.199.10.10:ftp

foo.bar.com:2000

To connect TNT, a remote console program is available: TNTC. It uses its own

configuration file tntc.ini, where the home directory and the socket to

connect is stored. At start TNTC sends the terminal type (environment entry

TERM) and the lines and columns of the display to TNT. Except for the number

of columns which are fixed to the value specified after 'input_linelen' in

tnt.ini, these values are taken to provide correct screen positioning and

attributing.

TNTC can be finished without terminating TNT by giving command QUIT. With

EXIT both TNT and TNTC will be terminated.

----------------------------------------------------------------------------

4. Description of configuration files

----------------------------------------------------------------------------

4.1. Main configuration file

The configuration of the program is done by the main configuration file.

This file called 'tnt.ini' is searched at the current directory or at the

home directory of the user (other filenames can be specified by using the -i

option at startup).

sample tnt.ini:

# defines if select() shall be used, normally 1, put to 0, if you are

# using old kernels (select() produces there a higher CPU-load).

use_select 1

# --------------------------------------------------------------------------

# 0 for real TNC at serial port, 1 for TFKISS on a UNIX-socket,

# 2 for TFKISS on other socket

soft_tnc 0

# serial port to which TNC is connected, UNIX-socket of TFKISS or

# other socket for TFKISS

device /dev/cua0

# lockfile for serial port or TFKISS

tnt_lockfile /usr/spool/uucp/LCK..cua0

# baudrate used, only used if TNC at serial port

speed 19200

# timinig parameters for interface to tfkiss (soft_tnc is 1)

# 1: fixed wait (10ms) after each hostmode-packet sent to tfkiss

fixed_wait 1

# if fixed_wait == 0, number of sent characters to tfkiss after which

# tnt will wait 10ms

amount_wait 20

# --------------------------------------------------------------------------

# number of channels of TNC

tnc_channels 10

# first channel with reduced backscroll buffer

r_channels 4

# --------------------------------------------------------------------------

# enable static huffman compression (//COMP)

tnt_comp 1

# packet length for transmission of files

file_paclen 255

# set to 1 to disconnect all channels on startup

disc_on_start 0

# --------------------------------------------------------------------------

# UNIX-user for remote permissions

remote_user guest

# If set to 1, new users will be created, if set to 0, all new users will

# be logged in as user defined by 'remote_user'.

unix_new_user 1

# lowest user-id for creation of new users

unix_first_uid 410

# group-id for new users

unix_user_gid 101

# --------------------------------------------------------------------------

# timeout packet assembly (in seconds) for SHELL/REDIR and interface

pty_timeout 2

# --------------------------------------------------------------------------

# set to 1 for request of R:-headers in boxlist read command

blist_add_plus 0

# --------------------------------------------------------------------------

# SSID of DPBox (A channel with this SSID must exist in TNC!)

tnt_box_ssid 7

# Call and SSID of DPBox (A channel with this Call and SSID

# must exist in TNC!)

# (If this parameter is used, 'tnt_box_ssid' will be ignored)

#tnt_box_call

# SSID of a node connect (not yet ready)

tnt_node_ssid 9

# Call and SSID of a node connect (not yet ready)

#tnt_node_call

# --------------------------------------------------------------------------

# main directory

tnt_dir /work/tnt/

# remote directory

remote_dir remote/

# ctext directory

ctext_dir ctext/

# directory for corrupt autobin-files

abin_dir abin/

# directory for uploads

upload_dir up/

# directory for downloads

download_dir down/

# directory for 7plus

tnt_7plus_dir 7plus/

# directory for YAPP

yapp_dir yapp/

# directory for executable programs

run_dir bin/

# home-dir for new users

unix_user_dir tntusers/

# dir for macro-texts

macrotext_dir macro/

# dir for box-broadcastfiles

tnt_bcnewmaildir bcast/newmail/

# dir for broadcastfiles

tnt_bcsavedir bcast/save/

# dir for temporary broadcastfiles

tnt_bctempdir /tmp/

# upfile

tnt_upfile tnt.up

# downfile

tnt_downfile tnt.dwn

# file containing process id

proc_file tnt.pid

# remote infofile

rem_info_file tntrem.inf

# remote helpfile

rem_help_file tntrem.hlp

# tnthelpfile

tnt_help_file tnt.hlp

# cookiefile

tnt_cookiefile /usr/games/fortunes/startrek

# namesfile

name_file_name names.tnt

# routesfile

route_file_name routes.tnt

# newsfile

news_file_name news.tnt

# connect text

tnt_ctextfile ctext.tnt

# logbook file

tnt_logbookfile log.tnt

# password file

tnt_pwfile pw.tnt

# sysop access files

tnt_sysfile sys.tnt

# calls with remote disabled

tnt_noremfile norem.tnt

# calls which do flexnet-linkquality-check

tnt_flchkfile flchk.tnt

# own call/SSID not allowed for xconnect

tnt_notownfile notown.tnt

# resync logfile

resy_log_file resy.log

# broadcast logfile

bcast_log_file bcast.log

# socket passwordfile

sock_passfile netpass.tnt

# file for keyboard macros

func_key_file fkeys.tnt

# file for extended remote commands

tnt_extremotefile extrem.tnt

# file for autostart on connect

tnt_autostartfile autostrt.tnt

# socket for digipoint box

box_socket /work/box/stat/socket

# directory for newmail

newmaildir newmail/

# file holding monitored folders

autobox_dir autobox.dir

# file for boxfile endings

tnt_boxender boxender.tnt

# file for f6fbb-definitions

f6fbb_box /work/box/system/f6fbb.box

# socket for tntnode (not yet ready)

node_socket /work/tntnode/socket

# socket for frontend

frontend_socket unix:tntsock

# --------------------------------------------------------------------------

# maximum length of input line

input_linelen 80

# set to 1 if insertmode shall be active after startup

insertmode 0

# maximum number of entries in heardlist

num_heardentries 100

# number of lines for backscroll

# command-screen

lines_command 50

# monitor-screen

lines_monitor 400

# input area of connect-screens

lines_input 20

# output area of connect-screens

lines_output 100

# input area of connect-screens (reduced backscroll)

lines_r_input 5

# output area of connect-screens (reduced backscroll)

lines_r_output 20

# input/output-lines ratio on real screen (connect)

scr_divide 5

# monitor lines on connect-screens

lines_moncon 0

# input area of mailbox-screen

lines_mbinput 10

# output area of mailbox-screen

lines_mboutput 200

# input/output-lines ratio on real screen (mailbox)

mbscr_divide 8

# input area of extended monitor screens

lines_xmon_pre 10

# output area of extended monitor screens

lines_xmon 100

# input/output-lines ratio on real screen (extended monitor)

xmon_scr_divide 5

# --------------------------------------------------------------------------

# 0: monochrom, 1: use color attributes if TERM = 'linux' or 'conXXX',

# otherwise use monochrom attributes and termcap, 2: use color attributes,

# 3: use color attributes and termcap if TERM = 'xterm'

color 1

# 0: don't use termcap, 1: use termcap

termcap 0

# 1: alternative channel status line

altstat 0

# --------------------------------------------------------------------------

# attributes for color

# normal characters

attc_normal 0x07

# characters in bottom statusline

attc_statline 0x10

# characters in monitor-headers

attc_monitor 0x06

# characters in channel statusline

attc_cstatline 0x1F

# control-characters

attc_controlchar 0x0F

# remote-answers

attc_remote 0x06

# special features

attc_special 0x01

# --------------------------------------------------------------------------

# attributes for monochrom

# normal characters

attm_normal 0x00

# characters in bottom statusline

attm_statline 0x08

# characters in monitor-headers

attm_monitor 0x10

# characters in channel statusline

attm_cstatline 0x10

# control-characters

attm_controlchar 0x10

# remote-answers and own transmitted text

attm_remote 0x10

# special features

attm_special 0x10

# --------------------------------------------------------------------------

# these values must remain unchanged using LINUX

# 1: terminal puts cursor to a new line after character in last column

auto_newline 0

# 1: don't display characters 128-160

supp_hicntl 0

# --------------------------------------------------------------------------

<EOF>

----------------------------------------------------------------------------

4.1.1. Serial and general configuration

'use_select'

Defines if select() shall be used, normally 1, put to 0, if you are

using old kernels (select() produces there a higher CPU-load)

'soft_tnc'

Defines if a real TNC (0) is connected or TFKISS (1,2).

'device'

Device, to which the TNC is connected, must be specified. If TFKISS is

used and soft_tnc is 1, device must contain the path and filename of

the UNIX-socket. If soft_tnc is 2, device must contain a general socket

description like explained in the Detailled description of the socket

servers.

'speed'

Baudrate of the serial port, only used if configured for a real TNC

'tnt_lockfile'

Filename to lock the port. Shall follow the usual conventions: for

device /dev/cua0 specify the lockfile /usr/spool/uucp/LCK..cua0. For

TFKISS use a pseudo-lockfile like /usr/spool/uucp/tfkiss or similar.

'fixed_wait'

Only valid if TFKISS is used: Normally set to 1, which means a fixed

wait of 10ms after each hostmode-packet sent to TFKISS. If 0,

'amount_wait' defines the waiting.

'amount_wait'

Only valid if TFKISS is used and 'fixed_wait' set to 0: Number of

characters sent to TFKISS after which TNT will wait 10ms (can be used

to optimize the timing).

'tnc_channels'

Number of channels of TNC.

'r_channels'

First channel with reduced backscroll buffer.

'tnt_comp'

If not 0, static huffman compression is activated. In this case the

maximum file packet length ('file_paclen') is 255 characters.

'file_paclen'

Defines the maximum length of a packet. Valid values are 20 up to 256

characters. If huffman compression is activated ('tnt_comp'), the

maxmimum value is 255 characters. The value can be changed by command

FPACLEN.

'disc_on_start'

If 1, all established connection are disconnected on startup of TNT, if

0, connections remain active.

'blist_add_plus'

If not set to 0, a '+' is appended to the boxlist read command. As a

result the mail will contain full R: lines.

'tnt_box_ssid'

If the SSID of the own call on the current channel is equal to this

SSID, DPBox is started on connect.

'tnt_box_call'

If the own call (plus SSID) on the current channel is equal to this

call (plus SSID), DPBox is started on connect. This parameter will

override 'tnt_box_ssid'. It can be used, if different callsigns with

the same SSID shall be used and DPBox shall be activated only for one

of the calls.

'tnt_node_ssid'

If the SSID of the own call on the current channel is equal to this

ssid, TNTNODE is started on connect. (function currently under

development).

'tnt_node_call'

If the own call on the current channel is equal to this call, TNTNODE

is started on connect. This parameter will override 'tnt_node_ssid'. It

can be used, if different callsigns with the same SSID shall be used

and TNTNODE shall be activated only for one of the calls. (function

currently under development).

----------------------------------------------------------------------------

4.1.2. Security

'remote_user'

User-Id for remote access.

If TNT was started by root, prior to every remote file command the user-id

is changed to restrict the file permissions of the remote user. If TNT was

started by a normal user, the user-id is not changed. Therefore a remote

user has the same file permissions as the user himself.

'unix_new_user'

1: A new user gets an entry in /etc/passwd and a directory is created.

0: A new user is logging in as specified in 'remote_user'.

'unix_first_uid'

First user-ID which will be taken for new users.

'unix_user_gid'

Group-ID used for new users.

----------------------------------------------------------------------------

4.1.3. Directories, Files and Sockets

The directory 'tnt_dir' must always contain the complete path. For all other

files and directories it depends on the first character of the path. If the

first character is a '/', then the path is taken as the complete path. If

the first character is not '/', 'tnt_dir' is put in front of the path.

4.1.3.1. Directories

'tnt_dir'

Directory which contains tnt-files

'remote_dir'

Directory used for remote access

'ctext_dir'

Directory used for personal connect texts

'abin_dir'

Directory for files of unsuccessful AutoBIN-transfers

'upload_dir'

Directory for sending files (SEND/SENDLOG/...) if no directory was

specified.

'download_dir'

Directory for receiving files (READ/LOGQSO/...) if no directory was

specified.

'yapp_dir'

Directory for receiving files using AUTOYAPP.

'tnt_7plus_dir'

Directory for receiving files using AUTO7PL.

'run_dir'

Directory for executable programs (//RUN).

'unix_user_dir'

Root-directory for directories of new users.

'macrotext_dir'

Directory for files containing text-macros (MSEND)

'tnt_bcnewmaildir'

Directory where received broadcast-files of type BBS will be stored.

'tnt_bcsavedir'

Directory where other received broadcast-files will be stored.

'tnt_bctempdir'

Directory for temporary files generated by broadcast receiver.

'newmaildir'

Directory where mails read by AUTOBOX and MONBOX are stored for DPBox

4.1.3.2. Files

'tnt_upfile'

Command script executed at startup of program

'tnt_downfile'

Command script executed before exit of program

'proc_file'

File which will contain the process ID of the running TNT process

'rem_info_file'

File transmitted if remote command //info is received

'rem_help_file'

File transmitted if remote command //help is received

'tnt_help_file'

File which is displayed if help screen is selected

'tnt_cookiefile'

File which contains database for fortune cookies

'name_file_name'

File for names database

'route_file_name'

File for routing database

'news_file_name'

File for news message, transmitted if //news is received or if %i macro

is found in a connect text file

'tnt_ctextfile'

File containing common connect text

'tnt_logbookfile'

File for logbook

'tnt_pwfile'

File for generation of passwords

'tnt_sysfile'

File for access restriction and sysop authentification

'tnt_noremfile'

File containing calls for which remote-commands are disabled

'tnt_flchkfile'

File containing calls which do flexnet-linkquality-check

'tnt_notownfile'

File containing call plus SSID which are not allowed for xconnect

'resy_log_file'

Logfile if a resync occured

'bcast_log_file'

Logfile for errors/warnings from broadcast receiver/transmitter

'sock_passfile'

Socket login database

'func_key_file'

Definition of keyboard macros

'tnt_extremotefile'

File defining extended remote commands

'tnt_autostartfile'

File defining autostart commands executed on connect

4.1.3.3. Sockets and Boxfiles

'box_socket'

Path and name of the UNIX-socket used for connection to DPBox.

'autobox_dir'

File containing boards which shall be monitored using MONBOX-feature.

'tnt_boxender'

File containing strings indicating end or abort of message (MONBOX and

AUTOBOX).

'f6fbb_box'

File containing strings which are used by the F6FBB-Box you are using.

This file is used by DPBox, too and shall be placed in the

DPBox-system-directory. A correct definition is needed for the

AUTOBOX-feature.

'node_socket'

Path and name of the UNIX-socket used for connection to TNTNODE

(currently under development).

'frontend_socket'

Name of the socket used by frontend TNTC if TNT is started as a daemon.

The syntax is identical to the socket definition in the detailled

description of the socket servers.

----------------------------------------------------------------------------

4.1.4. Lines of virtual screens

'input_linelen'

Maximum length of input line at which additional characters are ignored

or wordwrap is executed. The value can be changed with command LINelen.

'insertmode'

Set to 1 if insertmode shall be active on all screens after startup.

'num_heardentries'

Number of entries in heardlist.

'lines_command'

Number of lines of the command screen.

'lines_monitor'

Number of lines of the monitor screen.

'lines_input'

Number of lines of the input part of the connect screen.

'lines_output'

Number of lines of the output part of the connect screen.

'lines_r_input'

Number of lines of the input part of the connect screen (reduced

backscroll buffer).

'lines_r_output'

Number of lines of the output part of the connect screen (reduced

backscroll buffer).

'scr_divide'

Input/output-lines ratio on real screen (connect). The value can be

changed with command CONDiv. Example: Real screen has 25 Lines,

scr_divide is 5. 25 Lines - 2 Statuslines = 23 lines for input/output.

23 / 5 = 4.6 -> 4 lines on input. 23 - 4 = 19 lines for output.

'lines_moncon'

Number of lines of monitor screen additionally displayed on the connect

screen. Set to 0, if no monitor lines shall be displayed. The value can

be changed with command MONLines (The lines of the real screen will be

reduced by the number of monitor lines, before the lines of input and

output part will be calculated using scr_divide)

'lines_mbinput'

Number of lines of the input part of the mailbox screen.

'lines_mboutput'

Number of lines of the output part of the mailbox screen.

'mbscr_divide'

Input/output-lines ratio on real screen (mailbox). The value can be

changed with command MBOXDiv.

'lines_xmon'

Number of lines of the output part of the extended monitor screen.

'lines_xmon_pre'

Number of lines of the input part of the extended monitor screen.

'xmon_scr_divide'

Input/output-lines ratio on real screen (extended monitor screen). The

value can be changed with command XMONDiv.

----------------------------------------------------------------------------

4.1.5. Display configuration

'color'

non color attributes (0), color attributes if LINUX- console (1) or

color attributes always (2)

'termcap'

use LINUX-console control codes (0) or /etc/termcap (1)

'altstat'

Set to 1 if you want to use the alternative statusline on

connect-screens.

if color == 1 and LINUX-console control codes are selected, but the TERM

entry in the environment is not "con..." or "linux", no color and termcap is

used.

----------------------------------------------------------------------------

4.1.6. Screen attributes

Depending on the selected mode (color) the color or monochrom attributes are

used. Important notice: The attributes for normal text and for

controlcharcters in the input field MUST be different, otherwise no

control-characters can be sent.

4.1.6.1. Color attributes

'attc_normal'

For normal text.

'attc_statline'

For main statusline at the bottom of the screen.

'attc_monitor'

For monitor headers.

'attc_cstatline'

For statusline of channel in connect screen.

'attc_controlchar'

For controlcharacters in the input field.

'attc_remote'

For text transmitted because of remote functions.

'attc_special'

For special features.

The attributes are coded in the following manner:

X    X    X    X    X    X    X    X

Blink BCo2 BCo1 BCo0 FCo3 FCo2 FCo1 FCo0

Background-      Foreground-

Colour           Colour

Blink:

0:off, 1:on

Background Color:

BCo2 BCo1 BCo0

Black     0    0    0

Red       0    0    1

Green     0    1    0

orange    0    1    1

blue      1    0    0

magenta   1    0    1

cyan      1    1    0

white     1    1    1

Foreground Color:

FCo3 FCo2 FCo1 FCo0

Black           0    0    0    0

Red             0    0    0    1

Green           0    0    1    0

Orange          0    0    1    1

Blue            0    1    0    0

Magenta         0    1    0    1

Cyan            0    1    1    0

White           0    1    1    1

Grey            1    0    0    0

Light red       1    0    0    1

Light green     1    0    1    0

yellow          1    0    1    1

Light blue      1    1    0    0

Light magenta   1    1    0    1

Light cyan      1    1    1    0

Light white     1    1    1    1

4.1.6.2. Monochrom attributes

'attm_normal'

For normal text.

'attm_statline'

For main statusline at the bottom of the screen.

'attm_monitor'

For monitor headers.

'attm_cstatline'

For statusline of channel in connect screen.

'attm_controlchar'

For controlcharacters in the input field.

'attm_remote'

For text transmitted because of remote functions.

'attm_special'

For special features.

The attributes are coded in the following manner:

X    X    X    X    X    X    X    X

Att5 Att4 Att3 Att2 Att1 Att0

Att0: Standout    (termcap so/se)

Att1: Underline   (termcap us/ue)

Att2: Blink       (termcap mb/me)

Att3: Reverse     (termcap mr/me)

Att4: Bold        (termcap md/me)

Att5: Halfbright  (termcap mh/me)

0: off, 1: on

----------------------------------------------------------------------------

4.1.7. Packet assembly timeout

'pty_timeout'

Time for packet assembly

All characters received by a shell or redirection are buffered. If the

buffer reaches the maximum packetsize (256 bytes) the buffer is transmitted.

After each new character pty_timeout seconds are waited for the next

character. If during this time no character is given, the buffer will be

sent even if the maximum packetsize is not reached.

----------------------------------------------------------------------------

4.1.8. Additional options

The following switches are available to customize TNT to a different

terminal or operating system. For use with linux, both must be set to 0.

'auto_newline'

Set to 1 if terminal performs a newline after last character in a line

'supp_hicntl'

Set to 1 if characters in range 128-160 shall be substituted by a

period (.)

----------------------------------------------------------------------------

4.2. TNC-configuration files

At the startup of the program a command script is executed. It can be used

to initialize some parameters in the TNC. The filename must be specified

after 'tnt_upfile' in the init file.

To set up different callsigns on specific channels the following command

sequence can be used:

...

CHANNEL 4

MYCALL DL4YBG-4

CHANNEL 5

MYCALL DL4YBG-5

CHANNEL 6

MYCALL DL4YBG-6

...

After the EXIT command is given another command script is executed before

the termination of the program. The filename must be specified after

'tnt_downfile' in the init file.

----------------------------------------------------------------------------

4.3. Cookie file

If fortune cookie sending is active (command COOKIE) and someone is

connecting to the station a randomly selected cookie out of the cookie file

will be sent.

The cookie file consists of cookie texts delimited by a line containg a '-'

or a '%' as the first character (the rest of the line will be ignored). The

minimal size of a cookie file is 2048 bytes.

The data files contained in the 'fortunes' package in most

Linux-distributions can be used.

The filename of the cookie file must be given after 'tnt_cookiefile' in the

init file.

----------------------------------------------------------------------------

4.4. Files for remote commands

If the remote command //help is received a file specified after

'rem_help_file' in the init file is transmitted. The file shall contain an

overview and an explanation of the possible remote commands.

If the remote command //info is received a file specified after

'rem_info_file' in the init file is transmitted. The file shall contain a

description of the station and the used equipment.

If the remote command //news is received a file specified after

'news_file_name' in the init file is transmitted. The file shall contain any

news about the station.

----------------------------------------------------------------------------

4.5. Files for connect text

If connect text sending is active (command CONTEXT) and someone is

connecting to the station a special connect text will be sent. The connect

text is fetched from the file specified after 'tnt_ctextfile'. Any macros

contained in the file will be expanded. It is possible to send several users

a personal connect text. To achieve this a file called <callsign>.ctx must

be put to the directory specified after 'ctext_dir', where <callsign> is the

call of the user.

----------------------------------------------------------------------------

4.6. Files for names database and routing scripts

----------------------------------------------------------------------------

4.6.1. Names database

For use with the connect text feature a names database is included. The

database itself is contained in the file specified after 'name_file_name'.

All entries in the names database use the following format: T>DL4YBG Mark

If the command NAME or the remote command //NAME is used to update a name,

the names database file will be updated.

----------------------------------------------------------------------------

4.6.2. Routing database

For routing scripts the routing database is contained in a file with a

similar layout like the names database. The filename of the routing database

is specified after 'route_file_name'.

Unlike the names database file, the routing database is not updated by tnt,

any updates must be done with an text editor.

The entries for routing scripts can be recursive, it is not needed to

specify the whole path for each callsign. The path is assembled using the

relevant entries in the database.

If you frequently change the frequency and the uplink digipeater, you may

want to specify for every frequency a different routing database. Therefore

<IF xxxxxx> and <END> is included. If for example you operate on 438.300

Mhz, you can specify all routing information valid only on this frequency

by:

<IF 438.300>

...

<END>

All routing information which is not surrounded by <IF xxxxxx> and <END> is

valid on all frequencies. The frequency you are operating on can be

specified using the command QRG.

The format of each database entry is as follows:

In front of the callsign a letter specifies the type of the station:

T : Normal user

N : Network node

All other characters are accepted. The only difference is the treatment of

the call in routing scripts.

The database is a text file containing lines like the following:

T>DL4YBG Mark

-> The name of the operator of station DL4YBG is Mark, no routing

information given or station can be reached directly.

T>DL7ZZZ Joe; N>DB0LUC T>DL7ZZZ

-> The name of the operator of station DL7ZZZ is Joe, the station can

be connected from digipeater DB0LUC.

N>DB0BLO Digi 9K6

-> The digipeater DB0BLO can be connected directly.

N>DB0BER Digi Tempelhof; N>DB0BLO N>DB0BER

-> The digipeater DB0BER can be connected from digipeater DB0BLO.

N>DB0LUC Digi Luckau; N>DB0BER F>DB0BLN N>DB0LUC

-> The digipeater DB0LUC can be connected from digipeater DB0BER via

DB0BLN.

----------------------------------------------------------------------------

4.7. User-Id's and security

For every user it is possible to log into the UNIX-system by using //SHELL

or if the operator uses the SHELL command for the channel. The callsign of

the connected station is used as the user-id. If there is no user-id

existing with this call sign and 'unix_new_user' is not set, the user-id

"guest" is used. Because at the moment there is no security for the

password, no passwords are used. If you don't use 'unix_new_user' you must

prepare the user-id's of the authorized users yourself. Therefore the

entries in /etc/passwd shall look like:

guest::407:109::/home/guest:

dl4ybg::405:101::/home/dl4ybg:

These user-id's must not appear in /etc/shadow (only if shadow passwords are

used).

Hint:

The user guest can be created using 'useradd -m guest', then

/etc/passwd must be adapted and the password-entry in /etc/shadow (only

if shadow passwords are used) must be erased. Try a login from a

virtual terminal prior to logging in via remote command.

IMPORTANT:

Check the file permissions on your system and use a password for root, now

you are not alone any longer!

----------------------------------------------------------------------------

4.8. Logfile for resynchronisation

If during operation a resynchronisation is frequently started, it might be

useful to track down the reason for it. This can be achieved by specifying

'resy_log_file' in the init-file. The full path to the file must be given.

If left blank no resync log file is written. Have a look at the size of the

resync file, it is advisable to remove it from time to time to regain disc

space.

----------------------------------------------------------------------------

4.9. File for keyboard macros

sample fkeys.tnt:

#

# Function-Key-File

#

# XX:text

# XX::command

#

# '*' at end sends CR after a text

#

01::send .signature

02:***end*

03::c db0abc

04::c db0zzz-8

08::logblist

09::blist

10::xblist

<EOF>

----------------------------------------------------------------------------

4.10. File for password generation

sample pw.tnt:

# sample password file

# all lines must use the following format:

# DIEBOX:

# <callsign> <password-type> <password-file>

# FLEXNET:

# <callsign> <password-type> <secret-number> [priv-string]

# THENET:

# <callsign> <password-type> <password-file> <flags> <priv-string>

# BAYCOM:

# <callsign> <password-type> <password-file> <flags> [priv-string]

# MD2:

# <callsign> <password-type> <password-file>

#

# DieBox DB0XXX

DB0XXX-3 DIEBOX /work/tnt/db0xxx_3.pw

# FlexNet DB0YYY

DB0YYY FLEXNET 02345

# FlexNet DB0YYZ-8 with different activation string

DB0YYZ-8 FLEXNET 02345 /SYSop

# TheNet DB0ZZZ

DB0ZZZ THENET /work/tnt/db0zzz.pw 0 SYS

# TheNet DB0ZZA-2 with 3 tries

DB0ZZA-2 THENET /work/tnt/db0zza_2.pw 1 SYS

# TheNet DB0ZZB with password hiding

DB0ZZB THENET /work/tnt/db0zzb.pw 2 SYS

# TheNet DB0ZZC-15 with password hiding and 3 tries

DB0ZZC-15 THENET /work/tnt/db0zzc_15.pw 3 SYS

# TheNet DB0ZZB-1 with perfect password hiding

DB0ZZB-1 THENET /work/tnt/db0zzb_1.pw 6 SYS

# TheNet DB0ZZC-14 with perfect password hiding and 3 tries

DB0ZZC-14 THENET /work/tnt/db0zzc_14.pw 7 SYS

# Baycom DB0ZZE-5

DB0ZZE-5 BAYCOM /work/tnt/db0zze_5.pw 0

# Baycom DB0ZZD with password hiding and different activation string

DB0ZZD BAYCOM /work/tnt/db0zzd.pw 2 SYS

# Baycom DB0ZZD-1 with perfect password hiding and different activation string

DB0ZZD-1 BAYCOM /work/tnt/db0zzd_1.pw 6 SYS

# MD2 password for DB0ZZF

DB0ZZF MD2 /work/tnt/db0zzf.pw

<EOF>

----------------------------------------------------------------------------

4.11. File for sysop authentification

sample sys.tnt:

# sample access restriction and sysop authentification file

# format:

# <callsign> <password-file> <sysop-level>

#

# DL7ZZZ with root access

DL7ZZZ /work/tnt/dl7zzz.pw 1

# DL7ZZY with no root access

DL7ZZY /work/tnt/dl7zzy.pw 0

<EOF>

----------------------------------------------------------------------------

4.12. File for remote commands disabling

sample norem.tnt:

# sample file for remote commands disabling

# format:

# <callsign with SSID>

#

DB0GR

DB0BLO-8

DB0JES-3

DB0TEM-8

<EOF>

----------------------------------------------------------------------------

4.13. File containing not own callsigns

sample notown.tnt:

# sample file for call/SSID-combinations which are not allowed as

# source callsign for xconnects

# format:

# <callsign with SSID>

#

DL4YBG

DL4YBG-7

<EOF>

----------------------------------------------------------------------------

4.14. File containing Flexnet digipeaters

sample flchk.tnt:

# sample file for ignoring Flexnet connection quality checks

# format:

# <callsign with SSID>

#

DB0BNO

DB0BLN

<EOF>

----------------------------------------------------------------------------

4.15. File for AX25-server access

sample netpass.tnt:

# netpass.tnt - file

# Syntax is:

# CALL PASSWORD AUTOCONNECT-CALL LEVEL

#

# Examples

#

# Next line sets user/pass=dl7zzz/geheim, no default connect, level=9

dl7zzz geheim none 9

#

# Next line sets user/pass=dl7zzy/secret, autoconnects

# to DB0ZZZ-8 always, and he has command-access-level=3

# This autoconnect-callsign means that dl7zzy CAN'T connect

# any other station than DB0ZZZ-8 ! - Excluding if he has level=9 !

dl7zzy secret DB0ZZZ-8 3

<EOF>

----------------------------------------------------------------------------

4.16. File for autostart on connect

sample autostrt.tnt:

# sample autostart file

# format:

# <callsign+SSID> <tnt command>

#

# connect dl4ybg-5 -> start convers user-client

dl4ybg-5 convers

# connect dl4ybg-12 -> get a channel status

dl4ybg-12 cs

# connect dl4ybg-13 -> get a unix shell

dl4ybg-13 shell

# connect dl4ybg-14 -> connect to convers-server

dl4ybg-14 socket unix:/tcp/sockets/convers

<EOF>

----------------------------------------------------------------------------

4.17. File for extended remote commands

sample extrem.tnt:

# sample file for extended remote commands

# format:

# <remote command> <access level> <significant chars> <tnt command>

#

# //SCONvers : connect convers (Unix-)socket

sconvers 0 4 socket unix:/tcp/sockets/convers

# //AXSERv : connect local ax25 server on port 2001 (only for sysop)

axserv 1 5 socket localhost:2001

<EOF>

----------------------------------------------------------------------------

4.18. Files for BBS features

sample boxender.tnt:

#

# additional strings to detect the end of a mailbox-message

#

# format: 'xy string'

# x: 1 = message complete, 2 = message aborted

# y: value from 1 to 4 (up to 4 strings)

#

# one prompt of baycom-box

11 Help Dir Read Erase Check REPly Send Alter Quit

# prompt of diebox, if message read cancelled

21 (H)elp (C)heck (L)ist (R)ead (S)end (E)rase (D)ir (U)sage (Q)uit

# flexnet-reconnect

22 *** reconnected to

<EOF>

sample f6fbb.box:

#

# Definition of F6FBB-Fileheaders for the MonitorCut-Feature of DP

# YOU WILL HAVE TO ALTER THESE SETTINGS!

# BUT: NEVER CHANGE THE ORDER OF THE DEFINITIONS!

# Unfortunately, the f6fbb-soft has two basic types of fileheaders

# The first one consists of 5 lines, the second (and newer one) of 7.

# Additionally those headers exist in many different lingual versions.

# Therefore you are obliged to setup this file for your personal use...

#

# If the Header Format of your local F6FBB doesn't match to these,

# please write me a msg.

#

# THE '#' STARTS A COMMENT LINE... THE FIRST FOUR UNCOMMENTED LINES

# ARE KEPT AS THE KEYWORDS, THE FOLLOWING 12 UNCOMMENTED LINES ARE

# THE MONTH IDENTIFIERS AS USED IN YOUR LOCAL F6FBB-BOX

#

#

# An old-style (and dutch language) fbb-header may look like this:

#

# Van : DC6OQ  voor IBM   @DL

# Type/Status : B$

# Datum/tijd  : 21-Mrt 13:55

# Bericht #   : 72618

# Titel       : hilfe aastor

#

# Now the definitions:

#

# first word in header (all signs until the senders callsign):

Van :

# third word in header

voor

#

#

# A new-style (and german language) fbb-header may look like this:

#

# Von        : DG8NBR

# Nach       : YAESU @EU

# Typ/Status : B$

# Datum      : 18-Jun 06:44

# BID/MID    : 17630BDB0BOX

# Meldung #  : 85385

# Titel      : info > FT 530

#

# Now the definitions:

#

# Denotator in first line:

#Von        :

# Denotator in second line:

#Nach       :

#

#

#

# A new-style (and german language) fbb-header may look like this:

#

#Von         : DG1RFG

#An          : WINGT@DL

#Typ/Status  : BF

#Datum/Zeit  : 28-Apr 08:26

#BID (MID)   : DQKBUMDB0BLO

#Msg #       : 457242

#Titel       : TNX ! WinGT und Passwort wer...

#

# Now the definitions:

#

# Denotator in first line:

Von         :

# Denotator in second line:

An          :

#

#

#

# month identifiers as used in your local f6fbb-bbs:

# (they depend on the selected language, too)

#

Jan

Feb

Mar

Apr

Mai

Jun

Jul

Aug

Sep

Okt

Nov

Dez

<EOF>

----------------------------------------------------------------------------

4.19. Configuration file for TNTC

sample tntc.ini:

# main directory

tntc_dir /work/tnt/

# socket for frontend

frontend_socket unix:tntsock

<EOF>

----------------------------------------------------------------------------

5. Additional Information

----------------------------------------------------------------------------

5.1. Options at startup

Usage : tnt [-i <init-file>] [-l <log-file>] [-u] [-d]

TNT has four startup options. A file after -i is taken as the init file and

a file after -l is opened on channel 1 equal to a LOGREC command. The -l

option is useful if the TNC holds data from the time without a terminal. If

the program was not normally ended and the serial port is locked, the

lockfile can be ignored with -u. If TNT shall be started as a daemon without

terminal the paramter -d must be specified. A connection to TNT can be

established with TNTC.

Usage : tntc [-i <init-file>] [-s <tnt-socket>]

TNTC has two startup options. A file after -i is taken as the init file. A

socket specified after -s will be used instead of the 'frontend_socket'

defined in tntc.ini.

----------------------------------------------------------------------------

5.2. Running under X11

TNT needs a XTERM or RXVT console for execution under X11. Therefore TNT can

be started from every XTERM or RXVT shell. As XTERM does not allow the use

of the <ALT> keysequences, I suggest the usage of RXVT, which will provide

the same handling as a linux-console. A sample script file 'xtnt' can be

used to create a new window for TNT. To change the parameters in 'xtnt' see

the manual entry for RXVT.

----------------------------------------------------------------------------

5.3. Porting of TNT

In config.h several options can be defined or undefined. Create a new entry

in config.h and a new makefile for your operating system. At the moment

Linux and Interactive UNIX (ISC) are supported. The codes generated by

function and special keys are defined in keys.h and must be adapted. Because

in most cases not all keys are listed in the /etc/termcap entry the keycodes

are defined in this way. If your porting was successful, please send me your

changes. They will be included then in the next release.

----------------------------------------------------------------------------

6. Credits and Contact

Thanks to Dieter, DK5SG / N0PRA for wampes. The source code contains many

interesting things and I took some parts of the shell-functions out of the

wampes-code.

Thanks to Joerg, DD8FR for providing the code for macro expansion in the

connect text and for using more than 80 characters per line.

Patrick (ex DL7AUC), who has ported TNT to ISC Unix and has contributed many

ideas and additional code for the socket feature, lost his life in an

accident. His work and support was very much appreciated and will be missed

very much. He will always be remembered.

Thanks to Joachim, DL8HBS (author of DigiPoint for ATARI) for providing me

with his sourcecode. Thanks also for the hours of discussing and planning

the porting of the box-part of DP to Linux and for the support during the

debugging sessions.

Thanks to Gert, DK3NY for the implementation of the password generation for

DIEBOX-BBS's and for several bug reports.

Thanks to Andreas, DK9HE for analysing and fixing the startup problem if TNT

was not invoked by root.

Thanks to Werner, DL4NER for code and ideas for the FlexNet password

generation.

Thanks to Mario, DL5MLO for providing the code for the alternative

statusline, the 'insertmode'-flag in tnt.ini, AUTO7PL and several bugfixes.

Thanks to Oliver, DL8NEG for code for the baycom password, for perfect

password hiding and for //RTT.

Thanks to Hansi, DL9RDZ for a bug report concerning unsuccessful socket

connect and the solution for it.

Thanks to Claudio, IW0FBB for code for the MD2 password.

Thanks to Martin, DL3FCC for providing a TeX-version of the documentation

(this was the trigger to leave the path of a clean ASCII-doc).

Thanks to Bruno, F1IRW and Daniel, F1RMB for the french translation of the

documentation.

Thanks to Matthias, DL2SUT for fixes in the daemon code.

The code for YAPP is based on work of Jeff Jacobsen, WA7MBL, Jonathan

Naylor, G4KLX and S N Henson.

Thanks to Sanne Graaf for code for //RTT and //RING.

Thanks to Gerd, DK3NZ and Jonny, DG4MMI for bug reports.

Thanks to all who have tested TNT and have given comments.

And last but not least thanks to Linus and all of the Linux-community for

giving us a superb operating system. Special thanks to Joseph H. Allen for

the JOE-editor and Dave Gillespie for the P2C Pascal to C translator.

If you have questions, comments or bug reports, just write a mail:

Ham Radio : DL4YBG @ DB0GR.#BLN.DEU.EU

Internet  : wahlm@berlin.snafu.de

73, Mark Wahl (DL4YBG)

----------------------------------------------------------------------------

A. Appendix

----------------------------------------------------------------------------

A.1. Static huffman compression table

This table was set up by DK4NB for SP 9.00

ASCII:  HUFFMAN                ASCII:  HUFFMANN

<  0>:  101010110010110        <128>:  100111111110110

<  1>:  101010101000010        <129>:  00100001

<  2>:  100111111100010        <130>:  100111111101110

<  3>:  101010110011110        <131>:  100111111100110

<  4>:  101010110001110        <132>:  111100000

<  5>:  101010101111110        <133>:  011111010111110

<  6>:  101010101110110        <134>:  011111010110110

<  7>:  101010101101010        <135>:  00111010010000

<  8>:  101010101011010        <136>:  101010110100000

<  9>:  1111001101             <137>:  101010110011100

< 10>:  101010101010010        <138>:  101010110011000

< 11>:  011111010110010        <139>:  101010110010100

< 12>:  101010101101110        <140>:  101010110010000

< 13>:  1111010                <141>:  101010110001100

< 14>:  101010101001010        <142>:  1010101101110

< 15>:  100111111111010        <143>:  101010110001000

< 16>:  100111111101010        <144>:  101010110000100

< 17>:  011111010111010        <145>:  101010110000000

< 18>:  101010110100010        <146>:  101010101111100

< 19>:  101010110011010        <147>:  101010101111000

< 20>:  101010110010010        <148>:  001110110

< 21>:  101010110001010        <149>:  101010101110100

< 22>:  101010110000010        <150>:  101010101110000

< 23>:  101010101111010        <151>:  101010101101100

< 24>:  101010101110010        <152>:  101010101101000

< 25>:  10101011011000         <153>:  10101011011001

< 26>:  101010110000110        <154>:  011111010011

< 27>:  101010101100110        <155>:  101010101100100

< 28>:  101010101011110        <156>:  101010101100000

< 29>:  101010101010110        <157>:  101010101011100

< 30>:  101010101001110        <158>:  101010101011000

< 31>:  101010101000110        <159>:  101010101010100

ASCII:  HUFFMAN                ASCII:  HUFFMANN

:  110                    <160>:  101010101010000

!:  001110101              <161>:  101010101001100

":  1010101111             <162>:  101010101001000

#:  00000000011            <163>:  101010101000100

$:  011111010100           <164>:  101010101000000

%:  10101011010111         <165>:  100111111111100

&:  000000000000           <166>:  100111111111000

':  10101011010110         <167>:  100111111110100

(:  011111000              <168>:  100111111110000

):  001111001              <169>:  100111111101100

*:  01111101000            <170>:  100111111101000

+:  000000000001           <171>:  100111111100100

,:  0001001                <172>:  100111111100000

-:  0111101                <173>:  011111010111100

.:  101110                 <174>:  011111010111000

/:  0011001                <175>:  0011101001011

0:  0010001                <176>:  011111010110100

1:  11110110               <177>:  011111010110000

2:  00111101               <178>:  101010110100011

3:  100111100              <179>:  101010110100001

4:  101111011              <180>:  101010110011111

5:  011111001              <181>:  101010110011101

6:  000000001              <182>:  101010110011011

7:  101010100              <183>:  101010110011001

8:  101111010              <184>:  101010110010111

9:  100111110              <185>:  101010110010101

::  00000011               <186>:  101010110010011

;:  1010101101111          <187>:  101010110010001

<:  1010101101101          <188>:  101010110001111

=:  001111000              <189>:  101010110001101

>:  001100000              <190>:  101010110001011

?:  00000000001            <191>:  101010110001001

ASCII:  HUFFMAN                ASCII:  HUFFMANN

@:  011111010101           <192>:  101010110000111

A:  0011100                <193>:  101010110000101

B:  0111100                <194>:  101010110000011

C:  1001110                <195>:  101010110000001

D:  1111111                <196>:  101010101111111

E:  001001                 <197>:  101010101111101

F:  10111100               <198>:  101010101111011

G:  00000010               <199>:  101010101111001

H:  00000001               <200>:  101010101110111

I:  11110001               <201>:  101010101110101

J:  00000000010            <202>:  101010101110011

K:  00110001               <203>:  101010101110001

L:  11110010               <204>:  101010101101111

M:  0011010                <205>:  101010101101101

N:  0001110                <206>:  101010101101011

O:  0001111                <207>:  101010101101001

P:  1011111                <208>:  101010101100111

Q:  10101011101            <209>:  101010101100101

R:  0011111                <210>:  101010101100011

S:  000101                 <211>:  101010101100001

T:  0011011                <212>:  101010101011111

U:  111100111              <213>:  101010101011101

V:  111100001              <214>:  101010101011011

W:  00100000               <215>:  101010101011001

X:  11111100               <216>:  101010101010111

Y:  1001111110             <217>:  101010101010101

Z:  100111101              <218>:  101010101010011

[:  101010111001           <219>:  101010101010001

\:  001110111              <220>:  101010101001111

]:  101010111000           <221>:  001110100111

^:  10101011010101         <222>:  101010101001101

_:  0011101001010          <223>:  101010101001011

ASCII:  HUFFMAN                ASCII:  HUFFMANN

`:  10101011010100         <224>:  101010101001001

a:  10100                  <225>:  001100001

b:  000110                 <226>:  101010101000111

c:  100110                 <227>:  101010101000101

d:  01110                  <228>:  101010101000011

e:  010                    <229>:  101010101000001

f:  000001                 <230>:  100111111111111

g:  101011                 <231>:  100111111111101

h:  111110                 <232>:  100111111111011

i:  0110                   <233>:  100111111111001

j:  0011101000             <234>:  100111111110111

k:  11111101               <235>:  100111111110101

l:  00101                  <236>:  100111111110011

m:  101100                 <237>:  100111111110001

n:  1000                   <238>:  100111111101111

o:  101101                 <239>:  100111111101101

p:  0001000                <240>:  100111111101011

q:  011111010010           <241>:  100111111101001

r:  11100                  <242>:  100111111100111

s:  10010                  <243>:  100111111100101

t:  11101                  <244>:  100111111100011

u:  00001                  <245>:  100111111100001

v:  11110111               <246>:  011111010111111

w:  1010100                <247>:  011111010111101

x:  011111011              <248>:  011111010111011

y:  1111001100             <249>:  011111010111001

z:  0111111                <250>:  011111010110111

{:  10101011010010         <251>:  011111010110101

|:  0011101001001          <252>:  011111010110011

}:  10101011010011         <253>:  011111010110001

~:  001110100110           <254>:  001110100100011

<127>:  100111111111110        <255>:  001110100100010

----------------------------------------------------------------------------